CWTS-TSD-CN-29.010 V3.5.0

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network;

Information element mapping between Mobile Station - Base Station System (MS - BSS) and Base Station System - Mobile-services Switching Centre (BSS - MSC);

Signalling procedures and the Mobile Application Part (MAP)

(Release 1999)

Keywords

UMTS, GSM, network, MAP, SS7, protocol

CWTS

Internet

http://www.cwts.org

Contents

Foreword	5

1	Scope	6

1.1	References	6

1.2	Abbreviations	7

2	Classification of interworking cases	7

2.1	Transparent procedures	7

2.2	Non-transparent procedures	7

3	Interworking in the MSC, Transparent case	8

3.1	General	8

3.2	Location area updating	10

3.3	Detach IMSI	11

3.4	Routeing area updating	12

3.5	Authentication	13

3.6	Retrieval of the IMSI from the MS	14

3.7	Reallocation of TMSI	14

3.8	Retrieval of the IMEI from the MS	15

3.9	Tracing subscriber activity	15

4	Non-transparent cases	16

4.1	General	16

4.2	Outgoing call set-up (MS originating call)	16

4.3	Incoming call set-up (MS terminating call)	20

4.4	Cipher mode setting	22

4.5	Inter-MSC Handover	22

4.5.1	Basic Inter-MSC Handover	22

4.5.2	Subsequent Inter-MSC Handover back to MSC-A	28

4.5.3	Subsequent Inter-MSC Handover to third MSC	32

4.5.4	BSSAP Messages transfer on E-Interface	35

4.5.5	Processing in MSC-B, and information transfer on E-interface	36

4.5.5.1	Encryption Information	36

4.5.5.2	Channel Type	37

4.5.5.3	Classmark	38

4.5.5.4	Downlink DTX-Flag	38

4.5.5.5	Priority	38

4.5.5.6	MSC/BSC-Invoke Trace Information Elements	39

4.5.5.7	LSA Identifier List	39

4.5.6	Overview of the Technical Specifications GSM interworking for the Inter-MSC Handover	39

4.6	Inter-MSC Handover (UMTS to GSM)	41

4.6.1	Basic Inter-MSC Handover	41

4.6.2	Subsequent Inter-MSC Handover from  3G-MSC-B back to MSC-A	46

4.6.3	Subsequent Inter-MSC Handover to third MSC	50

4.6.4	BSSAP Messages transfer on E-Interface	54

4.6.5	Cause Code Mapping	54

4.7	Inter-MSC Handover (GSM to UMTS)	55

4.7.1	Basic Inter-MSC Handover	55

4.7.2	Subsequent Inter-MSC Handover from MSC-B back to 3G_MSC-A	61

4.7.3	Subsequent Inter-MSC Handover to third MSC	65

4.7.4	BSSAP Messages transfer on E-Interface	67

4.7.4.1	Assignment	67

4.7.4.2	Cipher Mode Control	68

4.7.4.3	Location Reporting Control	69

4.7.5	Processing in 3G_MSC-B, and information transfer on E-interface	69

4.7.5.1	Encryption Information	69

4.7.5.2	Channel Type	69

4.7.5.3	Classmark	70

4.7.5.4	Priority	70

4.7.5.5	MSC-Invoke Trace Information Elements	70

4.7.6	Cause Code Mapping	70

4.8	Inter-MSC Relocation	73

4.8.1	Basic Inter-MSC Relocation	73

4.8.2	Subsequent Inter-MSC Relocation back to 3G_MSC-A	78

4.8.3	Subsequent Inter-MSC Relocation to third MSC	82

4.8.4	RANAP Messages transfer on E-Interface	85

4.8.5	Processing in 3G_MSC-B, and information transfer on E-interface	86

4.8.5.1	Integrity Protection Information	86

4.8.5.2	Encryption Information	87

4.8.5.3	RAB Parameters	87

4.8.5.4	Channel Type	87

4.8.6	Overview of the Technical Specifications 3GPP interworking for the Inter-MSC Relocation	88

Annex A (informative): Change history	90

# Foreword

This Technical Specification (TS) has been produced by the cwts.

The present document specifies Information element mapping between Mobile Station - Base Station System (MS - BSS) and Base Station System - Mobile-services Switching Centre (BSS - MSC) Signalling procedures and the Mobile Application Part (MAP) within the digital cellular telecommunications system.

The contents of the present document are subject to continuing work within the CWTS and may change following formal CWTS approval. Should the CWTS modify the contents of the present document, it will be re-released by the CWTS with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to CWTS for information;

2	presented to CWTS for approval;

3	or greater indicates CWTS approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The scope of the present document is:

i)	to provide a detailed specification for the interworking between information elements contained in layer 3 messages sent on the MS-MSC interface (Call Control and Mobility Management parts of GSM 04.08) and parameters contained in MAP services sent over the MSC-VLR interface (GSM 09.02) where the MSC acts as a transparent relay of information;

ii)	to provide a detailed specification for the interworking between information elements contained in BSSMAP 	messages sent on the BSC-MSC interface (GSM 08.08) and parameters contained in MAP services sent over the 	MSC-VLR interface (GSM 09.02) where the MSC acts as a transparent relay of information;

iii)	to provide a detailed specification for the interworking between information elements contained in BSSMAP 	messages (GSM 08.08) and RANAP (25.413)

iv)	to provide a detailed specification for the interworking as in i) and ii) above when the MSC also processes the information.

Interworking for supplementary services is given in GSM 09.11. Interworking for the short message service is given in GSM 03.40 and GSM 04.11. Interworking between the call control signalling of GSM 04.08 and the PSTN/ISDN is given in GSM 09.03, GSM 09.07 and GSM 09.08. Interworking between the 'A' and 'E' interfaces for inter-MSC handover signalling is given in GSM 09.07 and 09.08.

## 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.905: "3G Vocabulary".

[2]	3GPP TS 23.009: "Handover procedures".

[3]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[4]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols-Stage 3".

[5]	3GPP TS 24.010: "Mobile radio interface layer 3  Supplementary services specification - General aspects".

[6]	3GPP TSº24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[7]	3GPP TS 25.413: "Iu interface RANAP signalling".

[8]	3GPP TS 27.001: " General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS)".

[9]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[10]	3GPP TS 29.007: "General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)".

[11]	3GPP TS 29.011: "Digital cellular telecommunications system (Phase 2+); Signalling interworking for supplementary services".

[12]	GSM 08.08: "Digital cellular telecommunications system (Phase 2+); Mobile Switching Centre - Base Station System (MSC - BSS) interface  Layer 3 specification".

[13]	GSM 09.03: "Digital cellular telecommunications system (Phase 2+); Signalling requirements on interworking between the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) and the Public Land Mobile Network (PLMN)".

[14]	GSM 09.08: "Digital cellular telecommunications system (Phase 2+); Application of the Base Station System Application Part (BSSAP) on the E-interface".

[15]	3GPP TS 29.108:  “Application of the Radio Access Network Application Part (RANAP) on the E-interface”

## 1.2	Abbreviations

Abbreviations used in the present document are listed in 3GPP TS 21.905.

# 2	Classification of interworking cases

## 2.1	Transparent procedures

The following MSC procedures require transparent mapping of BSSAP information elements into MAP parameters and vice versa (see GSM 09.02 for definitions and the use of the procedures):

-	update location area;

-	detach IMSI;

-	forward new TMSI;

-	provide IMSI;

-	obtain IMEI;

-	check IMEI;

-	authenticate;

-	trace subscriber activity.

## 2.2	Non-transparent procedures

Procedures in this class require processing in the MSC and information element mapping. These procedures include those related to:

-	outgoing call set-up;

-	incoming call set-up;

-	handover;

-	cipher mode setting.

# 3	Interworking in the MSC, Transparent case

## 3.1	General

When the MSC receives a forward message from the BSS (possibly forwarded transparently from the MS), it will invoke the desired MAP service and establish a cross reference between the BSSAP procedure and the MAP procedure in order to return the result of the operation to the BSS (which may forward it transparently to the MS. The cross reference is deleted when the MSC terminates the MAP procedure.

Positive or negative results of the MAP procedure are returned in the appropriate BSSAP message.

The parameters of the forward BSSAP message are mapped by a one-to-one mapping into the parameters of the MAP service. However, in some cases parameters received on the radio path may be suppressed at the MSC because they are related to another protocol entity, e.g. information related to RR-management may be included in MM-management messages. Similarly, parameters received in the (positive) MAP service response are mapped one-to-one into parameters of the corresponding backward BSSAP message.

A negative outcome, as carried in various MAP services (MAP specific service response, MAP_U_ABORT, MAP_P_ABORT, MAP_NOTICE and premature MAP_CLOSE, see GSM 09.02 for definitions) is mapped into a cause value in the required backward BSSAP message. In this case several negative results of MAP may be mapped into the same BSSAP cause value, i.e. without discrimination between these negative results.

NOTE:	For O & M purposes, the MAP procedure entity in the MSC may require a more detailed overview of negative results than the MS.

These principles are illustrated in figure 1.

04.08 (08.08)                                MAP service

forward message                                request

------------>                                ----------->

+-----------+                                +---------+

|information|                                |parameter|

|  element  |                                |         |

+-----------+           one-to-one           +---------+

+----->-------------------------------->----+

mapping

MAP service

positive ack                                  response

<------------                               <-----------

+-----------+                                +---------+

|information|                                |parameter|

|  element  |                                |         |

+-----------+           one-to-one           +---------+

+-----<--------------------------------<----+

mapping

negative

negative cause                                response

<------------                               <-----------

+-----------+                                +---------+

|   cause   |                                |  cause  |

+-----------+    one-to-one or many-to-one   +---------+

+-----<--------------------------------<----+

mapping

Figure 1: Illustration of mapping principles in the MSC

For each of the transparent operations listed in subclause 2.1, the following format is used to show the mapping.

----------------------------------------------------------------

| 04.08 or 08.08                  09.02           |Notes

--------┼-------------------------------------------------┼-----

Forward | MS/BSS to MSC                 MSC to VLR        |

message |   message name              MAP service request |

| information element 1  <--->  parameter 1       |

| information element 2  <--->  parameter 2       |

--------┼-------------------------------------------------┼-----

Positive| MSC to MS/BSS                  VLR to MSC       |

result  |   message name               positive response  |

| information element 1  <--->  parameter 1       |

| information element 2  <--->  parameter 2       |

--------┼-------------------------------------------------┼-----

Negative| MSC to MS/BSS                  VLR to MSC       |

result  |   message name               negative response  |

|     cause 1            <--->     cause 1        |

|     cause 2            <--->     cause 2        |

|     cause 3            <--->  MAP_U/P_ABORT     |

|     cause 3            <--->   MAP_NOTICE       |

|     cause 3            <--->    MAP_CLOSE       |

--------┴-------------------------------------------------┴-----

Equivalent mapping principles apply for operations invoked by the VLR towards the BSS/MS. However, negative results are generally not received from the BSS/MS but are generated in the MSC. Therefore, for such operations the interworking for negative results is not normally shown.

## 3.2	Location area updating

---------------------------------------------------------------

|   08.08/04.08                 09.02            |Notes

--------┼------------------------------------------------┼-----

Forward | COMPLETE LAYER 3 INFO     MAP_UPDATE_LOCATION_ |

message | (LOCATION UPDATING          AREA request       |

|    REQUEST)                                    |

|                                                |

|    Location area id         Previous LA Id     |

|    Mobile identity          IMSI or TMSI       |

|    Mobile station                              |

|       classmark 1                  -           |  4

|    Ciphering key            CKSN               |

|      seq number                                |

|    Location update          Location update    |

|      type                     type             |  3

| Cell identifier             Target LA Id       |  1

| Chosen channel                     -           |

--------┼------------------------------------------------┼-----

Positive| DTAP (LOCATION            MAP_UPDATE_LOCATION  |

results | UPDATING ACCEPT)            AREA response      |

|                                                |

|    Location area identity          -           |

|    Mobile identity                 -           |  5

|    Follow on proceed               -           |

--------┼------------------------------------------------┼-----

Negative| DTAP (LOCATION            MAP_UPDATE_LOCATION  |

results | UPDATING REJECT)            AREA response      |

|                                                |

|    IMSI unknown in HLR    Unknown subscriber   |	 6

|    Network failure        Unknown LA           |  2

|                           Roaming not allowed: |

|    PLMN not allowed          PLMN not allowed  |

|    LA not allowed            LA not allowed    |

|    Roaming not               National Roaming  |

|    allowed in this LA            not allowed   |

|    PLMN not allowed          Operator          |

|                              determined barring|

|    Illegal MS             Illegal subscriber   |

|    Illegal ME             Illegal equipment    |

|    Network failure        System Failure       |

|    Network failure        Unexpected data value|

|    Network failure        MAP_U/P_ABORT        |

|    Network failure        MAP_NOTICE           |

|    Network failure        MAP_CLOSE            |

--------┴------------------------------------------------┴-----

NOTE 1:	The Target LA Id parameter is derived by the MSC from the Cell identifier information element.

NOTE 2:	The Unknown LA error is only generated as a result of incorrect information being inserted by the MSC or BSS.

NOTE 3:	This parameter can be used by the VLR to decide whether (e.g.) Authentication or IMEI checking is needed.

NOTE 4	As the mobile station classmark (1 or 2) is received by the MSC at the establishment of every RR connection, this information need not be stored in the VLR, but it is stored in the MSC as long as the RR connection exists.

NOTE 5	The mobile identity is inserted by the MSC if it is received in a MAP_FORWARD_NEW_TMSI service. If a TMSI is included, the MS should respond with a TMSI REALLOCATION COMPLETE message.

NOTE 6	The HLR shall also send this error if there is an error in the type of subscription (i.e. VLR requests service for a GPRS only subscriber).

## 3.3	Detach IMSI

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | IMSI DETACH              MAP_DETACH_IMSI       |

message |    INDICATION               request            |

|                                                |

| Mobile identity             IMSI or TMSI       |

|                                                |

| Mobile Station                                 |

|   classmark 1                 -                |

--------┼------------------------------------------------┼-----

Positive|                                                |

result  |                                                |  1

--------┼------------------------------------------------┼-----

Negative|                                                |

result  |                                                |

--------┴------------------------------------------------┴-----

NOTE 1:	The forward message is not acknowledged.

Depending on the state of the MS, the IMSI DETACH INDICATION may be carried in either a DTAP message or a BSSMAP COMPLETE LAYER 3 INFORMATION message.

## 3.4	Routeing area updating

---------------------------------------------------------------

|       04.08                   09.02            |Notes

--------┼------------------------------------------------┼-----

Forward | GMM (ROUTEING AREA        MAP_UPDATE_GPRS    _ |

message |     UPDATE REQUEST)         LOCATION request   |

|                                                |

|     MS classmark 1               -             |

|     MS classmark 4               -             |

|     GPRS Ciphering               -             |

|       key seq number                           |

|     Mobile station               IMSI          |

|       identity                                 |

|     Old routeing area            -             |

|       identification                           |

--------┼------------------------------------------------┼-----

Positive| GMM (ROUTEING AREA        MAP_UPDATE_GPRS      |

results |     UPDATE ACCEPT)          LOCATION response  |

|                                                |

|    Routeing area                 -             |

|       identification                           |

|    Mobile station                -             |  1

|       identity                                 |

| C  Mobile station                -             |	 2

| C  Reject: IMSI unknown          -             |  3

|            in HLR                              |

| C  Reject: MSC temporarily       -             |  4

|            not reacheable                      |

--------┼------------------------------------------------┼-----

Negative| GMM (ROUTEING AREA        MAP_UPDATE_GPRS      |

results |     UPDATE REJECT)          LOCATION response  |

|                                                |

|    Network failure               -             |  5

|    GPRS services             Unknown HLR       |

|       not allowed in                           |

|       this PLMN                                |

|    GPRS services          Unknown subscriber   |  6

|       not allowed       (no GPRS subscription) |

|    GPRS services and      Unknown subscriber   |  7

|       non GPRS services    (IMSI unknown)      |

|       not allowed                              |

| C  GPRS services          Unknown subscriber   |  8

|       not allowed       (no GPRS subscription) |

| C  GPRS services and      Unknown subscriber   |  9

|       non-GPRS services    (IMSI unknown)      |

|       not allowed                              |

|    MS identity cannot            -             |  10

|       be derived by                            |

|       the network                              |

|                           Roaming not allowed: |

|    GPRS services not         PLMN not allowed  |

│       allowed in this                          │

│       PLMN                                     │

|    LA not allowed                -             |

|    Roaming not allowed           -             |

|       in this LA                               |

|    GPRS services not         Operator          |

|       allowed in this        determined barring|

│       PLMN                                     │

|    Illegal MS                    -             |

|    Illegal ME                    -             |

|    Network failure        System Failure       |

|    Network failure        Unexpected data value|

|    Network failure        MAP_U/P_ABORT        |

|    Network failure        MAP_NOTICE           |

|    Network failure        MAP_CLOSE            |

--------┴------------------------------------------------┴-----

NOTE 1:	The mobile station identity is inserted by the SGSN if the SGSN wants to deallocate or re-allocate a P-TMSI. If the SGSN wants to deallocate the P-TMSI it shall include the IMSI. If the SGSN wants to re-allocate the P-TMSI it shall include the new P-TMSI. If a P-TMSI is included, the MS shall respond with a ROUTEING AREA UPDATE COMPLETE message.

NOTE 2:	The mobile station identity is inserted by the SGSN if it is received in a BSSAP+ LOCATION UPDATE ACCEPT message from the VLR. If a TMSI is included, the MS shall respond with a ROUTEING AREA UPDATE COMPLETE message. Only used in the Combined Routeing and Location Area procedure.

NOTE 3:	This reject cause is inserted on the positive response by the SGSN if the SGSN receives a BSSAP+ LOCATION UPDATE REJECT message from the VLR indicating in the reject cause IMSI unknown in HLR. Only used in the Combined Routeing and Location Area procedure.

NOTE 4:	This reject cause is inserted on the positive response by the SGSN if the SGSN does not receive any response from the VLR to a previous BSSAP+ LOCATION UPDATE REQUEST message. Only used in the Combined Routeing and Location Area procedure.

NOTE 5:	The Unknown RA error is only generated as a result of incorrect information being inserted by the BSS.

NOTE 6:	The HLR shall send Unknown subscriber with diagnostic value No GPRS subscription if the HLR indicates that there is an error in the type of subscription (i.e. SGSN requests service for a non-GPRS only subscriber).

NOTE 7:	The HLR shall send Unknown subscriber with diagnostic value IMSI unknown if the HLR indicates that the IMSI provided by the SGSN is unknown.

NOTE 8:	The HLR shall send Unknown subscriber with diagnostic value No GPRS subscription if the HLR indicates that there is an error in the type of subscription (i.e. SGSN requests service for a non-GPRS only subscriber). Used in the Combined Routeing and Location Area procedure.

NOTE 9:	This reject cause is inserted if the SGSN receives a MAP GPRS UPDATE LOCATION negative response message indicating IMSI unknown. Used in the Combined Routeing and Location Area procedure.

NOTE 10:	This reject cause is inserted if the SGSN does not receive any response from the old SGSN to a previous SGSN CONTEXT REQUEST message.

## 3.5	Authentication

The message flow for the authentication procedure is shown in figure 2.

MS                        MSC                                VLR

MAP_AUTHENTICATE request

AUTHENTICATION REQUEST    <----------------------------------

<-----------------------

AUTHENTICATION RESPONSE

----------------------->        MAP_AUTHENTICATE response

---------------------------------->

or

MAP_U/P_ABORT

---------------------------------->

Figure 2: Authentication operation

The MSC can only act on a MAP_AUTHENTICATE request if an RR connection exists with the MS. If such a connection does not exist, the MSC shall terminate the MAP procedure with a MAP_U_ABORT. The same applies if the MS does not respond to an AUTHENTICATION REQUEST message.

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | AUTHENTICATION REQUEST   MAP_AUTHENTICATE      |

message |                             request            |

|                                                |

| RAND                     RAND                  |

|                                                |

| Ciphering key seq        CKSN                  |

| number                                         |

--------┼------------------------------------------------┼-----

Backward| AUTHENTICATION REQUEST   MAP_AUTHENTICATE      |

result  |                             response           |

|                                                |

| SRES                     SRES                  |

--------┴------------------------------------------------┴-----

If the SRES parameter does not match the value stored in the VLR, then the ongoing MAP procedure shall be terminated with a cause 'illegal subscriber'. This shall cause the MSC to send an AUTHENTICATION REJECT message.

## 3.6	Retrieval of the IMSI from the MS

The VLR may request open identification of an MS with a MAP_PROVIDE_IMSI request.

The mapping of information elements is as follows:

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | IDENTITY REQUEST         MAP_PROVIDE_IMSI      |

message |                             request            |

|     Identity type                              |

|     set to: IMSI                               |  1

--------┼------------------------------------------------┼-----

Backward| IDENTITY RESPONSE        MAP_PROVIDE_IMSI      |

result  | Mobile Identity (IMSI)      response           |

--------┴------------------------------------------------┴-----

NOTE 1:	The INVOKE does not carry any parameters. The identity type is inferred from the invoke name.

The MSC shall return a MAP_PROVIDE_IMSI response with user error "absent subscriber" if:

-	there is no RR connection with the MS when the MAP service request is received;

-	there is no response from the MS.

## 3.7	Reallocation of TMSI

This operation is invoked by the VLR. The MAP_FORWARD_NEW_TMSI request contains the new TMSI which is forwarded to the MS in the TMSI REALLOCATION COMMAND. When the MS acknowledges the receipt of the new TMSI, the MSC will return a MAP_FORWARD_NEW_TMSI response to the VLR.

If there is no radio connection to the MS when the MSC receives the MAP service request, the MSC shall ignore the message.

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | TMSI REALLOCATION     MAP_FORWARD_NEW_TMSI     |

message |    COMMAND               request               |

|                                                |

| Mobile identity       TMSI                     |

|                                                |

| Location area                                  |

| identification               -                 |

--------┼------------------------------------------------┼-----

Backward| TMSI REALLOCATION     MAP_FORWARD_NEW_TMSI     |

result  |  COMPLETE                response              |

--------┴------------------------------------------------┴-----

## 3.8	Retrieval of the IMEI from the MS

The VLR may use the MAP_OBTAIN_IMEI service to request the MS to supply its IMEI , or may use the MAP_CHECK_IMEI service to request the MSC to check the MS's IMEI. For either MAP service the BSSAP signalling is the same.

The mapping of information elements is as follows:

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward |                      (MAP_CHECK_IMEI request   |

message | IDENTITY REQUEST     (   or                    |

|                      (MAP_OBTAIN_IMEI request  |

|    Identity type                               |

|    set to: IMEI                                |  1

--------┼------------------------------------------------┼-----

Backward|                      (MAP_CHECK_IMEI response  |

result  | IDENTITY RESPONSE    (   or                    |

|                      (MAP_OBTAIN_IMEI response |

|                                                |

| Mobile Identity          IMEI                  |  2

|    (IMEI)                                      |

--------┴------------------------------------------------┴-----

NOTE 1:	The MAP service request does not carry any parameters. The identity type is inferred from the service name.

NOTE 2:	If the MAP_CHECK_IMEI service was used, the MSC also returns the equipment status to the VLR in the MAP_CHECK_IMEI response, after a successful dialogue with the EIR using the IMEI received from the MS.

The MSC shall terminate the MAP dialogue with the VLR using a MAP_U_ABORT if:

-	there is no RR connection with the MS when the MAP service request is received;

-	there is no response from the MS.

NOTE:	The MSC can also obtain the IMEI from a phase 2 MS by including appropriate information in the BSSMAP Cipher Mode Command.

## 3.9	Tracing subscriber activity

The VLR may request the MSC and/or BSS to record data about the current transaction with an MS.

---------------------------------------------------------------

|  08.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | MSC INVOKE TRACE      MAP_TRACE_SUBSCRIBER_    |

message |                          ACTIVITY request      |

|                                                |

| Trace type               Trace type            |

| TriggerId                    -                 |

| Trace reference          Trace reference       |

| TransactionId                -                 |

| Mobile identity(IMSI)    IMSI                  |  1

| Mobile identity(IMEI)    IMEI                  |  1

| OMCId                    OMCId                 |

--------┼------------------------------------------------┼-----

Backward| none                  none                     |

result  |                                                |

--------┴------------------------------------------------┴-----

NOTE 1:	The VLR may provide either an IMSI or IMEI, but not both.

# 4	Non-transparent cases

## 4.1	General

For interworking other than the mapping of information fields, see GSM 09.08.

## 4.2	Outgoing call set-up (MS originating call)

Figure 3 shows those elements of a call set-up sequence which require interworking between BSSAP and MAP. BSSAP messages which do not require interworking with MAP are not shown.

MS                        MSC                                VLR

CM SERVICE REQUEST

----------------------->   MAP_PROCESS_ACCESS_REQUEST request

---------------------------------->

+--------------------------+

|         Possibly         |

| identification procedure/|

| authentication procedure |

+--------------------------+

MAP_SET_CIPHERING_MODE request

<----------------------------------

(Note 1)

MAP_PROCESS_ACCESS_REQUEST response

CM SERVICE ACCEPT      <---------------------------------

<-----------------------         positive result (Note 1)

(Note 2)

CIPHER MODE COMMAND

<-----------------------

(Note 2)

CIPHER MODE COMPLETE

----------------------->

MAP_FORWARD_NEW_TMSI request

TMSI REALLOCATION COMMAND  <----------------------------------

<-----------------------                 (Note 3)

TMSI REALLOCATION COMPLETE

----------------------->      MAP_FORWARD_NEW_TMSI response

---------------------------------->

MAP_PROCESS_ACCESS_REQUEST response

CM SERVICE REJECT      <---------------------------------

<-----------------------     negative result, MAP_U/P_ABORT,

(Note 4)                  MAP_NOTICE, MAP_CLOSE

SETUP (Note 5)

----------------------->     MAP_SEND_INFO_FOR_OUTGOING_CALL

---------------------------------->

request

MAP_COMPLETE_CALL request

CALL PROCEEDING       <----------------------------------

<-----------------------

MAP_SEND_INFO_FOR_OUTGOING_CALL

RELEASE COMPLETE       <----------------------------------

<-----------------------        response, MAP_U/P_ABORT,

(Note 6)                 MAP_NOTICE, MAP_CLOSE

Figure 3: Part of outgoing call set-up sequence

NOTE 1:	If the MSC received a MAP_SET_CIPHERING_MODE request, it stores it until it receives the MAP_PROCESS_ACCESS_ REQUEST response.

NOTE 2:	CM SERVICE ACCEPT is sent only if the ciphering procedure is not invoked.

NOTE 3:	TMSI reallocation need not be sequenced with other messages, but should be sent after ciphering.

NOTE 4:	CM SERVICE REJECT is sent as a result of a user error parameter in the MAP_PROCESS_ACCESS_REQUEST response, or termination of the MAP dialogue.

NOTE 5:	The SETUP message is sent after the MS has either received a CM SERVICE ACCEPT or sent a CIPHER MODE COMPLETE.

NOTE 6:	RELEASE COMPLETE is sent as a result of a user error parameter in the MAP_SEND_INFO_FOR_OUTGOING_CALL response, or termination of the MAP dialogue.

The procedure can be considered in two parts: the handling of the CM SERVICE REQUEST and the handling of the SETUP request.

The procedure is initiated by the MS sending a CM SERVICE REQUEST message. The MSC will forward the service request to the VLR in the MAP_PROCESS_ACCESS_REQUEST request. The VLR may then invoke other operations, e.g. authentication and identification. These operations are defined in subclauses 3.4 and 3.5.

If there is a positive outcome for the CM SERVICE REQUEST procedure, the VLR always sends a MAP_PROCESS_ACCESS_REQUEST response. If the request is for a first MM-connection and ciphering is required, the MAP_PROCESS_ACCESS_REQUEST response is preceded by a MAP_SET_CIPHERING_MODE request. In this case the MSC sends a CIPHER MODE COMMAND towards the MS. The interworking for cipher mode setting is described in subclause 4.4. If the request is for an additional MM-connection or for a first MM-connection where ciphering is not required, then the positive MAP_PROCESS_ACCESS_ REQUEST response causes the MSC to send a CM SERVICE ACCEPT message to the MS. After cipher mode setting has been completed or the CM SERVICE ACCEPT message has been returned, the MS will send the SETUP (or EMERGENCY SETUP) message and information retrieval takes place as shown.

A negative outcome for the MAP_PROCESS_ACCESS_REQUEST procedure can be signalled by a MAP_PROCESS_ACCESS_REQUEST response containing a user error parameter, or by terminating the MAP dialogue between the MSC and the VLR.

A positive outcome for the call setup procedure is indicated by a MAP_COMPLETE_CALL request from the VLR to the MSC, which causes the MSC to send a CALL PROCEEDING message towards the MS.

A negative outcome for the call setup procedure can be signalled by a MAP_SEND_INFO_FOR_INCOMING_CALL response or by terminating the dialogue between the MSC and the VLR.

Information element mapping is required between the messages:

-	CM SERVICE REQUEST to MAP_PROCESS_ACCESS_REQUEST request;

-	SETUP to MAP_SEND_INFO_FOR_OUTGOING CALL request;

-	MAP_SEND_INFO_FOR_OUTGOING_CALL response, MAP_U/P_ABORT, MAP_NOTICE or premature MAP_CLOSE to RELEASE COMPLETE or CM SERVICE REJECT.

The information contained in the MAP_COMPLETE_CALL request is not transmitted on the radio interface but is used in the MSC for connecting the call.

The conversion of information elements is as follows:

---------------------------------------------------------------

|  08.08/04.08             09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | COMPLETE LAYER 3 INFO   MAP_PROCESS_ACCESS_    |

| (CM SERVICE REQUEST)       REQUEST request     |

|                                                |

|    CM Service type      CM Service type        |  1

|    Ciphering key        CKSN                   |

|       sequence number                          |

|    Mobile identity      TMSI or IMSI or IMEI   |

|    Mobile station                              |

|       Classmark 2             -                |

|                                                |

| Cell identifier         Current LA Id          |  4

| Chosen channel                -                |

|         -               Access Connection      |

|                            Status              |  3

--------┼------------------------------------------------┼-----

Positive| DTAP(CM SERVICE ACCEPT) MAP_PROCESS_ACCESS_    |

result  |                            REQUEST response    |  2

--------┼------------------------------------------------┼-----

Negative| DTAP(CM SERVICE REJECT) MAP_PROCESS_ACCESS_    |

result  |                            REQUEST response    |

|                                                |

|    IMSI unknown in VLR     Unidentified        |

|                               Subscriber       |

[       |    Requested service     ???????               |

|       option not                               |

|       subscribed                               |]

|    Illegal ME              Illegal equipment   |

|    Network failure         System failure      |

|    Network failure      MAP_U/P_ABORT          |

|    Network failure      MAP_NOTICE             |

|    Network failure      MAP_CLOSE              |

--------┼------------------------------------------------┼-----

| DTAP(AUTHENTICATION     MAP_PROCESS_ACCESS_    |

|    REJECT)                 REQUEST response    |

|                                                |

|                            Illegal subscriber  |

--------┴------------------------------------------------┴-----

NOTE 1:	Indicates, in this case, a mobile originating call establishment or an emergency call establishment.

NOTE 2:	The CM SERVICE ACCEPT is sent when the ciphering procedure is not invoked.

NOTE 3:	Indicates whether or not an RR-connection exists and whether or not ciphering has been started.

NOTE 4:	The Current LA Id parameter is derived by the MSC from the Cell identifier information element.

---------------------------------------------------------------

|  04.08                   09.02                 |Notes

--------┼------------------------------------------------┼-----

Forward | SETUP                   MAP_SEND_INFO_FOR_     |

message |                         OUTGOING_CALL request  |

|                                                |

| BC repeat indicator              -             |

| Bearer capability 1              -             |  3

| Bearer capability 2              -             |  3

| Calling party subaddress         -             |

| Called party BCD number     Called Number      |

| Called party subaddress          -             |

| LLC repeat indicator             -             |

| Low layer compatibility I        -             |

| Low layer compatibility II       -             |

| HLC repeat indicator             -             |

| High layer compatibility i       -             |

| High layer compatibility ii      -             |

|      -                      Bearer service     |  3

|      -                      Teleservice        |  3

| Facility                         -             |  1

|      -                      CUG index          |  4

|      -                      Suppress pref CUG  |  4

|      -                      Suppress CUG OA    |  4

| User-user                        -             |

| SS version                       -             |

| CLIRO flag                       -             |

--------┼------------------------------------------------┼-----

Positive|                                                |

result  |                                                |  2

--------┼------------------------------------------------┼-----

Negative| RELEASE COMPLETE         MAP_SEND_INFO_FOR_    |

result  |                          OUTGOING_CALL response|

|                                                |

| TS GSM 04.10             Call Barred           |

|                             Barring Service    |

|                                Active          |

| Operator determined      Call Barred           |

|    barring                  Operator Determined|

|                                Barring         |

| Network out of order     Data Missing          |

| Network out of order     Unexpected Data Value |

| Network out of order     System Failure        |

| Bearer capability        Bearer service not    |

|    not authorized           provisioned        |

| Bearer capability        Teleservice not       |

|    not authorized           provisioned        |

| [User not member of CUG] CUG reject            |

|                                                |

| Network out of order     MAP_U/P_ABORT         |

| Network out of order     MAP_NOTICE            |

| Network out of order     MAP_CLOSE             |

--------┴------------------------------------------------┴-----

NOTE 1:	If the Facility IE contains CUG information, the CUG information is transferred to the VLR in the MAP_SEND_INFO_FOR_OUTGOING_CALL service; any other information contained in a Facility IE is transferred to the VLR in a MAP Supplementary Services related service.

NOTE 2:	The call setup parameters retrieved from the VLR are not sent to the MS. The parameters are carried in the MAP_COMPLETE_CALL service.

NOTE 3:	The bearer capabilities can be used to derive the bearer/tele service.

NOTE 4:	CUG information is derived from the contents of the Facility IE.

## 4.3	Incoming call set-up (MS terminating call)

Figure 4 shows those elements of the procedure which require interworking between MAP and GSM 04.08 procedures.

MS                        MSC                                VLR

+--------------------------+

|       Info retrieval     |

+--------------------------+

MAP_PAGE request or

PAGE REQUEST         <----------------------------------

<-----------------------         MAP_SEARCH_FOR_MS request

(Note 1)

PAGING RESPONSE

----------------------->   MAP_SEARCH_FOR_MS response (Note 2)

---------------------------------->

MAP_PROCESS_ACCESS_REQUEST request

---------------------------------->

+--------------------------+

|         Possibly         |

| authentication procedure |

+--------------------------+

MAP_SET_CIPHERING_MODE request

<----------------------------------

(Note 3)

MAP_PROCESS_ACCESS_REQUEST response

CIPHER MODE COMMAND     <---------------------------------

<-----------------------         positive result (Note 4)

(Note 3)

CIPHER MODE COMPLETE

----------------------->

MAP_FORWARD_NEW_TMSI request

TMSI REALLOCATION COMMAND  <----------------------------------

<-----------------------                 (Note 5)

TMSI REALLOCATION COMPLETE

----------------------->      MAP_FORWARD_NEW_TMSI response

---------------------------------->

MAP_COMPLETE_CALL request

SETUP            <----------------------------------

<-----------------------

MAP_SEND_INFO_FOR_INCOMING_CALL

RELEASE COMPLETE       <---------------------------------

<-----------------------        response negative result,

(Note 6)           MAP_U/P_ABORT,MAP_NOTICE, MAP_CLOSE

Figure 4: Incoming call set-up

NOTE 1:	If an MM connection already exists, the PAGE REQUEST is not sent. If the call can be accepted, the MSC sends a MAP_PROCESS_ACCESS_REQUEST request in response to the MAP_PAGE request. If the call cannot be accepted the MSC sends a MAP_PAGE response containing the error 'busy subscriber'.

NOTE 2:	Sent only if MAP_SEARCH_FOR_MS was used.

NOTE 3:	Needed only if a ciphered MM-connection does not exist already.

NOTE 4:	If the MSC received a MAP_SET_CIPHERING_MODE request, it stores it until it receives the MAP_PROCESS_ACCESS_ REQUEST response.

NOTE 5:	TMSI reallocation need not be sequenced with other messages, but should be sent after ciphering.

NOTE 6:	RELEASE COMPLETE is sent as a result of a user error parameter in the MAP_SEND_INFO_FOR_OUTGOING_CALL response, or termination of the MAP dialogue.

The paging procedure is controlled by the VLR. It may be followed by authentication (subclause 3.4), ciphering (subclause 4.4) and reallocation of TMSI(subclause 3.6). The SETUP message is sent when the MAP_COMPLETE_CALL request is received.

Normally there is no interworking between the MAP_COMPLETE_CALL request and the SETUP message. However, the MAP_COMPLETE_CALL request may contain a bearer service indication which will be used to establish the bearer capabilities at the MSC. The interworking between the MAP_PAGE request or MAP_SEARCH_FOR_MS request and the BSSMAP PAGING REQUEST message is as follows:

---------------------------------------------------------------

|  08.08/04.08              09.02                |Notes

--------┼------------------------------------------------┼-----

Forward | PAGING REQUEST       MAP_PAGE request or       |

message |                      MAP_SEARCH_FOR_MS request |

|                                                |

| IMSI                    IMSI                   |

| TMSI                    TMSI                   |  1

| Cell identifier         Stored LA Id           |

|       list                                     |

--------┼------------------------------------------------┼-----

Backward| COMPLETE LAYER 3 INFO    MAP_PROCESS_ACCESS_   |

message | (PAGING RESPONSE)           REQUEST request    |

|                                                |

|          -                  CM service type    |  2

|    Ciphering key            CKSN               |

|       sequence number                          |

|    Mobile identity          TMSI or IMSI       |

|    Mobile station                              |

|       classmark 2                  -           |

| Cell Identifier             Current LA Id      |  3

|          -                  Access connection  |

|                                status          |

| Chosen channel                     -           |

--------┴------------------------------------------------┴-----

NOTE 1:	If TMSI is included, the TMSI is used as the mobile identity in the GSM 04.08 PAGE REQUEST message, otherwise the IMSI is used as the mobile identity.

NOTE 2:	In this case the MAP CM service type is set to 'mobile terminating call'.

NOTE 3:	The Target LA Id parameter is derived by the MSC from the Cell identifier information element.

## 4.4	Cipher mode setting

The interworking is as follows:

---------------------------------------------------------------

|  08.08                  09.02                  |Notes

--------┼------------------------------------------------┼-----

Forward | CIPHER MODE COMMAND    MAP_SET_CIPHERING_MODE  |

|                           request              |

|                                                |

| Cipher mode setting       Ciphering mode       |

| Encryption information    Kc                   |  1

--------┼------------------------------------------------┼-----

Positive| CIPHER MODE COMPLETE   None                    |

result  |                                                |

--------┼------------------------------------------------┼-----

Negative| CIPHER MODE REJECT     None                    |

result  |                                                |

--------┴------------------------------------------------┴-----

NOTE 1:	The key Kc is passed through the BSS to the BTS, but is not passed to the MS.

## 4.5	Inter-MSC Handover

The general principles of the handover procedures are given in GSM 03.09. GSM 09.10 gives the necessary information for interworking between the GSM 08.08 handover protocol and the GSM 09.02 MAP protocol.

### 4.5.1	Basic Inter-MSC Handover

When a Mobile Station is handed over between two MSCs, the establishment of a connection between them (described in GSM 03.09) requires interworking between A-Interface and E-Interface.

The signalling at initiation, execution, completion of the Basic Inter-MSC handover procedure is shown in figures 5 to 10 with both possible positive or negative outcomes.

Additionally figures 5b and 5c show the possible interworking when trace related messages are transparently transferred on the E-Interface at Basic Inter-MSC Handover initiation.

BSS-A         MSC-A                     MSC-B

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request                  | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 5a: Signalling for Basic Inter-MSC Handover initiation (no trace related messages transferred)

BSS-A         MSC-A                     MSC-B

|BSC INVOKE TRACE                         |

|-------------->|                         |

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request(*)               | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

|               |                         |                   |

|               |                         |BSC INVOKE TRACE   |

|               |                         |---------------->(**)

Figure 5b: Signalling for Basic Inter-MSC Handover initiation (BSC invoke trace message transferred)

(*):	In that case, HANDOVER REQUEST and BSC INVOKE TRACE messages are included within the BSS-apdu parameter.

(**):	BSC INVOKE TRACE is forwarded to BSS-B if supported by MSC-B.

BSS-A         MSC-A                     MSC-B

|             (*)                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request(**)              | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

|               |                         |                   |

|               |                         |MSC INVOKE TRACE   |

|               |                         |--------------->(***)

Figure 5c: Signalling for Basic Inter-MSC Handover initiation (MSC invoke trace message transferred)

(*):	Tracing invocation has been received from VLR.

(**):	In that case, HANDOVER REQUEST and MSC INVOKE TRACE messages are included within the BSS-apdu parameter.

(***):	MSC INVOKE TRACE is forwarded to BSS-B if supported by MSC-B.

Possible Positive outcomes:

a)	successful radio resources allocation and handover number allocation (if performed):

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE HANDOVER    |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued and successful handover number allocation (if performed). Later successful radio resources allocation indication:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

HANDOVER COMMAND| SIGNALLING request      |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 6: Signalling for Basic Inter-MSC Handover execution (Positive outcomes)

Possible Negative outcomes:

c)	user error detected, or handover number allocation unsuccessful (if performed), or component rejection or dialogue abortion performed by MSC-B:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |MAP PREPARE HANDOVER response                |

|               |negative result, MAP CLOSE                   |

|               |<------------------------|                   |

|               |MAP U/P-ABORT            |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

d)	radio resources allocation failure:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

|               | SIGNALLING request      |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

f)	unsuccessful handover execution (Reversion to the old channel):

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|                         |                   |

|FAILURE        |                         |                   |

|               |MAP U -ABORT             |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |                   |

Figure 7: Signalling for Basic Inter-MSC Handover execution (Negative outcomes)

NOTE:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER COMPLETE  |

|               |                         |<------------------|

|               |MAP SEND END SIGNAL request                  |

|               |<------------------------|                   |

|CLEAR sCOMMAND |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 8: Signalling for Basic Inter-MSC Handover completion

Positive outcome

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               |MAP SEND END SIGNAL      |                   |

|               |------------------------>|                   |

|               | response                |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         | (Note 1)          |

Figure 9: Signalling for Basic Inter-MSC Handover completion (Positive outcome)

Negative outcome

BSS-A         MSC-A                     MSC-B              BSS-B

|               |                         |                   |

|               | MAP U/P -ABORT          |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |                   |

Figure 10: Signalling for Basic Inter-MSC Handover completion (Negative outcome)

NOTE:	From interworking between MAP and BSSMAP point of view.

The handover procedure is normally triggered by BSS-A by sending a HANDOVER REQUIRED message on A-Interface to MSC-A. The invocation of the Basic Inter-MSC handover procedure is performed and controlled by MSC-A. The sending of the MAP Prepare-Handover request to MSC-B is triggered in MSC-A upon receipt of the HANDOVER REQUIRED message. For compatibility reason, the cell identity of the cell where the call is to be handed over in MSC-B area, provided in the HANDOVER REQUIRED message, is mapped into targetCellId MAP parameter and the HANDOVER REQUEST message is encapsulated in the bss-APDU MAP parameter of the Prepare-Handover MAP request. MSC-B can invoke another operation towards the VLR-B (allocation of the handover number described in GSM 09.02).

Additionally, if tracing activity has been invoked, the trace related messages can be transferred on the E-Interface encapsulated in the bss-APDU MAP parameter of the Prepare-Handover Request. If transferred, one complete trace related message at a time shall be included in the bss-APDU MAP parameter after the HANDOVER REQUEST message.

The interworking between Prepare Handover and HANDOVER REQUIRED is as follows:

----------------------------------------------------------------

|   08.08                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER REQUIRED   MAP PREPARE HANDOVER request|

message |                                                 |

|                            -ho-NumberNotRequired| 1

|  BSSMAP information        -targetCellId        |

|         elements           -bss-APDU(           |   2

|                            HANDOVER REQUEST,    |

|                            BSC INVOKE TRACE     |   3

|                         or MSC INVOKE TRACE)    |

--------┼-------------------------------------------------┼-----

Positive|                    MAP PREPARE HANDOVER response|

result  |                                                 |   4

|                             -handover number    |

|                             -bss-APDU(          |

|                              QUEUING INDICATION |

|                              or HANDOVER REQUEST|

|                              ACKNOWLEDGE or     |

|                              HANDOVER FAILURE)  |

--------┼-------------------------------------------------┼-----

Negative| HANDOVER REQUIRED REJECT    MAP PREPARE HANDOVER|   5

result  |                                                 |

| equipment failure            System Failure     |

| equipment failure            No Handover Number |

|                                 available       |

| equipment failure            UnexpectedDataValue|

| equipment failure            Data Missing       |

|                                                 |

| equipment failure            MAP CLOSE          |

| equipment failure            MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The ho-NumberNotRequired parameter is included by MSC-A, when MSC-A decides not to use any circuit connection with MSC-B. No handover number shall be present in the positive result. Any negative response from MSC-B shall not be due to handover number allocation problem.

NOTE 2:	The process performed on the BSSMAP information elements received in the HANDOVER REQUIRED message is described in the GSM Recommendation 08.08.

NOTE 3:	The process performed on the BSSMAP information elements received in the MSC or BSC INVOKE TRACE message is described in subclause 4.5.6.6.

NOTE 4:	The response to the Prepare-Handover request can include in its bss-APDU parameter, identifying the GSM-08.06 protocol, either a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, MSC-A shall wait for the radio resources allocation response from MSC-B, transmitted to MSC-A as described in subclause 4.5.4.

In the second case, the positive result triggers in MSC-A the sending on A-Interface of the HANDOVER COMMAND.

In the third case, the positive result triggers in MSC-A one of the following:

-	another handover attempt is initiated by MSC-A;

-	optionally the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of GSM 09.10 and lies in GSM 08.08).

NOTE 5:	The possible sending of the HANDOVER REQUIRED REJECT message is described in GSM 08.08.

The interworking between Send End Signal and HANDOVER COMPLETE in MSC-B is as follows:

----------------------------------------------------------------

|   08.08                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER COMPLETE   MAP SEND END SIGNAL request |

message |                                                 |

|                               -bss-APDU(        |

|                               HANDOVER COMPLETE)|

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| CLEAR COMMAND       MAP SEND END SIGNAL response|

result  |  -Call Control release                          |  1

--------┼-------------------------------------------------┼-----

Negative| CLEAR COMMAND                                   |

result  |  -Call Control release        MAP CLOSE         |  2

|  -Call Control release        MAP U/P -ABORT    |

|                                                 |

NOTE 1:	The positive empty result triggers the clearing of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B. If a circuit connection is used between MSC-A and MSC-B, the 'Call Control release' clearing cause shall only be given to BSS-B when MSC-B has received a clearing indication on its circuit connection with MSC-A.

NOTE 2:	The abortion of the dialogue or the rejection of the component triggers in MSC-B the clearing of its circuit connection with MSC-A, if any, of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B.

The interworking between Send End Signal and CLEAR COMMAND in MSC-A is as follows:

----------------------------------------------------------------

|   09.02                          08.08          |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP SEND END SIGNAL          CLEAR COMMAND      |

message | response                                        |

|          -bss-APDU(           - Handover        |

|          HANDOVER COMPLETE)        Successful   |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

The interworking between HANDOVER FAILURE in case of reversion to old channel of the MS and User Abort in MSC-A is as follows:

----------------------------------------------------------------

|   08.08                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER FAILURE                MAP U -ABORT    |

message |                                                 |

|  - Reversion to old                             |

|    channel                                      |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

### 4.5.2	Subsequent Inter-MSC Handover back to MSC-A

When a Mobile Station is being handed over back to MSC-A, the procedure (described in GSM 03.09) requires interworking between A-Interface and E-Interface.

The signalling at initiation, execution and completion of the Subsequent Inter-MSC handover procedure is shown in figures 11 to 15.

BSS-A         MSC-B                     MSC-A

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE SUBSEQUENT   |

|REQUIRED       |------------------------>|

|               |HANDOVER request         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 11: Signalling for Subsequent Inter-MSC Handover back to MSC-A initiation

Possible Positive outcomes:

a)	successful radio resources allocation:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued. Later successful radio resources allocation indication:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP FORWARD ACCESS      |                   |

|               |<------------------------|                   |

|               | SIGNALLING request      |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

Figure 12: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution (Positive outcome)

Possible Negative outcomes:

c)	user error detected, or component rejection or dialogue abortion performed by MSC-A:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|HANDOVER REQUIRED  response negative result                  |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

d)	component rejection or dialogue abortion performed by MSC-A:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |MAP CLOSE, MAP U/P ABORT |                   |

|               |<------------------------|                   |

|CLEAR COMMAND  |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

e)	radio resources allocation failure:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         | HANDOVER FAILURE  |

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|HANDOVER REQUIRED   HANDOVER response    |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

f)	radio resources allocation queued. Later unsuccessful radio resources allocation:

BSS-A         MSC-B                     MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               |    HANDOVER response    |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               | MAP FORWARD ACCESS      |                   |

|               |<------------------------|                   |

|HANDOVER REQUIRED   SIGNALLING request   |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

Figure 13: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution
(Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

BSS-B         MSC-A                     MSC-B              BSS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

Figure 14: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion
(Successful completion of the procedure)

NOTE:	Positive outcome case shown in figure 9.

BSS-B         MSC-A                     MSC-B              BSS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |                   |

|               |MAP U/P -ABORT           |                   |

|               |<------------------------|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |(Note 1)           |

Figure 15: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion (Unsuccessful completion of the procedure)

NOTE 1:	Abnormal end of the procedure which triggers the clearing of all resources in MSC-B.

The interworking between Prepare Subsequent Handover and HANDOVER REQUIRED is as follows:

----------------------------------------------------------------

|   08.08                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward |HANDOVER REQUIRED MAP PREPARE SUBSEQUENT HANDOVER|

message |                              request            |  1

|                                                 |

|                              -target MSC number |

| BSSMAP information           -targetCellId      |

|        elements              -bss-APDU(         |

|                               HANDOVER REQUEST) |

--------┼-------------------------------------------------┼-----

Positive|HANDOVER REQUIRED MAP PREPARE SUBSEQUENT HANDOVER|

result  |                             response            |  2

|                             -bss-APDU(          |

|                              QUEUING INDICATION |

|                              or HANDOVER REQUEST|

|                              ACKNOWLEDGE or     |

|                              HANDOVER FAILURE)  |

--------┼-------------------------------------------------┼-----

Negative| HANDOVER REQUIRED REJECT  MAP PREPARE SUBSEQUENT|  3

result  |                           HANDOVER response     |

| equipment failure            Unknown MSC        |

| equipment failure            Subsequent Handover|

|                                         Failure |

| equipment failure            UnexpectedDataValue|

| equipment failure            Data Missing       |

|                                                 |

| CLEAR COMMAND                                   |

|                                                 |

| equipment failure            MAP CLOSE          |

| equipment failure            MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The processing performed on the BSSMAP information elements received in the HANDOVER REQUIRED message is out of the scope of the present document. The target MSC number is provided to MSC-A by MSC-B based on the information received from BSS-B.

NOTE 2:	The response to the Prepare-Subsequent-Handover request can include in its bss-APDU parameter, identifying the GSM-0806 protocol, either a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, MSC-B shall wait for the radio resources allocation response from MSC-A, transmitted to MSC-B as described in subclause 4.5.4.

In the second case, the positive result triggers in MSC-B the sending on A-Interface of the HANDOVER COMMAND.

In the third case, the positive result triggers in MSC-B one of the following:

-	another handover attempt is initiated by MSC-B;

-	optionally the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of GSM 09.10 and lies in GSM 08.08).

NOTE 3:	The possible sending of the HANDOVER REQUIRED REJECT message is described in GSM 08.08.

The interworking between Send End Signal Result and HANDOVER COMPLETE in MSC-A is as follows:

----------------------------------------------------------------

|   08.08                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER COMPLETE    MAP SEND END SIGNAL        |

message |                               response          |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                          MAP U/P -ABORT         |  1

|                                                 |

NOTE 1:	The abortion of the dialogue ends the handover procedure with MSC-B.

### 4.5.3	Subsequent Inter-MSC Handover to third MSC

When a Mobile Station is being handed over to a third MSC, the procedure (described in GSM 03.09) does require one specific interworking case in MSC-A (figure 20) between E-Interface from MSC-B and E-Interface from MSC-B' other than the combination of the ones described in the subclause 4.5.1 and 4.5.2.

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |

|----------->|MAP PREPARE SUSEQUENT |                |

|REQUIRED    |--------------------->|                |

|            |HANDOVER request      |MAP PREPARE     |

|            |                      |--------------->|

|            |                      |HANDOVER request|

|            |                      |                |+-------+

|            |                      |                ||Possib.|

|            |                      |                ||Alloc. |

|            |                      |                ||of ho. |

|            |                      |                ||number |

|            |                      |                || VLR-B |

|            |                      |                |+-------+

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |-------->|

|            |                      |                |REQUEST  |

|            |                      |                |

Figure 16: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') initiation

Possible Positive outcomes:

a)	successful radio resources allocation:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|    response    |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

|            |                      |                |         |

b)	radio resources allocation queued and successful handover number allocation, if performed. Later successful radio resources allocation indication:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|   response     |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |MAP FORWARD ACCESS    |SIGNALLING request        |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 17: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution
(Positive outcome)

Possible Negative outcomes:

c)	user error detected, or component rejection or dialogue abortion performed by MSC-B':

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |response negative result  |

|            |                      |MAP CLOSE       |         |

|            |                      |<---------------|         |

|            |                      |MAP U/P -ABORT  |         |

|            |MAP PREPARE SUBSEQUENT|                |         |

|            |<---------------------|                |         |

|            |HANDOVER response negative             |         |

| HANDOVER   |result                |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

d)	radio resources allocation failure:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |                      |SIGNALLING request        |

|            |MAP FORWARD ACCESS    |                |         |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

Figure 18: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution (Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

Positive outcome:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |COMPLETE |

|            |                      |                |         |

|            |                      |MAP SEND END SIGNAL       |

|            |                      |<---------------|         |

|            | MAP SEND END SIGNAL  |                |         |

|            |<---------------------|                |         |

|            | response             |                |         |

| CLEAR      |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 19: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Successful completion of the procedure)

Negative outcome:

BSS-A      MSC-B                  MSC-A            MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |    BSS-B'

|----------->|                      |                |         |

|FAILURE     |MAP PROCESS ACCESS    |                |         |

|            |--------------------->|                |         |

|            |SIGNALLING request (Note 1)            |         |

|            |                      |                |         |

|            |                      |MAP U -ABORT    |         |

|            |                      |--------------->|         |

|            |                      |                |CLEAR    |

|            |                      |                |-------->|

|            |                      |                |COMMAND  |

|            |                      |                |         |

Figure 20: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Unsuccessful completion of the procedure)

NOTE 1:	Specific interworking case detailed below.

The specific interworking case in MSC-A compared to the subclauses 4.5.1 and 4.5.2 occurs between HANDOVER FAILURE encapsulated in a Process Access Signalling from MSC-B and the abortion of the dialogue with MSC-B' in the case of a reversion to old channel of the MS:

----------------------------------------------------------------

|   09.02                          09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PROCESS-SIGNALLING                          |

message |         request                                 |

|                                                 |

|         -bss-APDU(               MAP U -ABORT   |  1

|         HANDOVER FAILURE)                       |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                  MAP U/P -ABORT |  2

|                                                 |

NOTE 1:	The abortion of the dialogue triggers in MSC-B' the clearing of the circuit connection with MSC-A, if any, and of the Resources between MSC-B' and BSS-B'.The abortion of the dialogue ends the handover procedure with MSC-B'.

NOTE 2:	The abortion of the dialogue ends the handover procedure with MSC-B.

### 4.5.4	BSSAP Messages transfer on E-Interface

The following mapping applies to the encapsulation performed in MSC-A.

----------------------------------------------------------------

|   04.08/08.08                    09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | BSSAP messages     MAP FORWARD ACCESS SIGNALLING|

message |                               request           |  1

|                                                 |

|                       -bss-APDU (BSSAP messages)|

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |  2

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP CLOSE         |

|                               MAP U/P -ABORT    |

|                                                 |

NOTE 1:	Complete BSSAP messages to be sent on MSC-B - BSS-B interface (BSSMAP or DTAP messages) are embedded into the bss-APDU parameter (see Annex A of GSM 08.08 for the description of the set of BSSMAP messages).

NOTE 2:	The Return Result does not apply. If MSC-B returns a message, this message will arrive in an Invoke: Process Access Signalling.

The following mapping applies to the encapsulation performed in MSC-B.

----------------------------------------------------------------

|   04.08/08.08                    09.02          |Notes

--------┼-------------------------------------------------┼-----

Forward | BSSAP messages     MAP PROCESS ACCESS SIGNALLING|

message |                                request          |  1

|                                                 |

|                       -bss-APDU (BSSAP messages)|

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |  2

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP CLOSE         |

| CLEAR COMMAND                                   |

|                                                 |

| equipment failure             MAP U/P -ABORT    |  3

|                                                 |

NOTE 1:	Complete BSSAP messages to be sent to MSC-A (BSSMAP or DTAP messages) are embedded into the bss-APDU parameter (see GSM 09.08 for the description of the set of BSSMAP messages).

NOTE 2:	The Return Result does not apply. If MSC-A returns a message, this message will arrive in an Invoke: Forward Access Signalling.

NOTE 3:	The abortion of the dialogue triggers the clearing of the circuit connection with MSC-A, if any, of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B. The clearing of the Radio Resources (the clearing indication received from BSS-B is transmitted to MSC-A) or the loss of the SCCP connection between MSC-B and BSS-B, triggers in MSC-B the abortion of the dialogue on the E-Interface and the clearing of the circuit connection with MSC-A, if any.

### 4.5.5	Processing in MSC-B, and information transfer on E-interface

The following parameters require processing (e.g. to store the parameter, to internally generate the parameter) in MSC-B. The relevant BSSMAP procedures are mentioned to ease the comprehension, their detailed description is the scope of GSM 08.08. Each BSSMAP message listed in GSM 09.08 being transferred on E-interface shall use the mechanisms given in subclause 4.5.4 and is described in GSM 08.08.

#### 4.5.5.1	Encryption Information

A sequence of possible encryption algorithms can be sent to a BSS in Cipher Mode Command or Handover Request. The BSS chooses one of the listed algorithms and reports this back to the MSC in Cipher Mode Complete or Handover Request Acknowledge respectively.

The list of algorithms, the ciphering key and the chosen algorithm shall be stored by MSC-B, and the chosen value sent to MSC-A.

Transfer of Information:

If ciphering has not been performed before Inter-MSC Handover, this will be controlled by MSC-A after the completion of Inter-MSC Handover.

Ciphering control towards MSC-B:

If Ciphering has been performed before Inter-MSC Handover:

-	in the Handover Request BSSMAP message (information included).

The Handover Request Acknowledge should in this case contain the indication of the chosen algorithm.

If Ciphering has NOT been performed before Inter-MSC Handover:

-	in the Cipher Mode Command procedure between MSC-A and MSC-B.

If the encryption algorithm is changed at an intra-BSS handover in BSS-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure.

If the encryption algorithm is changed at an intra-MSC handover in MSC-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure which shall be initiated by MSC-B on reception from BSS-B of the Handover Complete message (the information being previously received in the Handover Request Acknowledge message).

Note also that the chosen encryption value may be contained in the BSSMAP Assignment Complete message. This may happen if the encryption value changes e.g. at a second assignment during a call (e.g. from TCH to SDCCH).

#### 4.5.5.2	Channel Type

Assignment Request and Handover Request (BSSMAP) may give the BSS a choice, in the same way as the Encryption Algorithm above. Depending on the Channel Type Info, the chosen channel may have impact on subsequent handovers, internal in MSC-B and inter-MSC controlled by MSC-A. Some values in channel Type Info indicate that if a particular channel once has been chosen, the same type must be used for the rest of the call.

The Channel Type, and the characteristics of the chosen channel shall be stored by MSC-B, and the Chosen Channel and/or Speech Version information elements transferred to MSC-A.

Transfer of Information:

Independently of the type of resource (Signalling only (e.g. SDCCH) or TCH) assigned to the MS, the Channel Type Information is transferred to MSC-B in:

-	the Handover Request BSSMAP message, and the Chosen Channel and/or Speech Version should be reported back to MSC-A in the Handover Request Acknowledge.

If a new type of resource is to be assigned after Inter-MSC Handover, this can be made with:

-	the BSSMAP Assignment procedure between MSC-A and MSC-B (Chosen Channel and/or Speech Version in Assignment Complete).

If the Channel Type (the chosen channel and/or chosen speech version) is changed at an intra-BSS handover in BSS-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure.

If the Channel Type (the chosen channel or chosen speech version) is changed at an intra-MSC handover in MSC-B this must be reported to MSC-A in:

-	the BSSMAP Handover Performed procedure which shall be initiated by MSC-B on reception from BSS-B of the Handover Complete message (the information being previously received in the Handover Request Acknowledge message).

#### 4.5.5.3	Classmark

This information shall be stored by MSC-B and might be received either from MSC-A, or from the MS when the MS initiates a Classmark Update.

Transfer of Information due to Classmark received from MSC-A:

This information shall be stored by MSC-B and is received:

-	in the Handover Request BSSMAP message.

If a new type of resource is to be assigned after Inter-MSC Handover, Classmark Information MAY be included:

-	in the BSSMAP Assignment procedure.

Transfer of Information, due to "Classmark Signalling Procedures".

This information shall be stored by MSC-B and can be received:

-	Due to a classmark update, either requested from MSC-A (Classmark Request, Classmark Update), or an MS-Initiated Classmark Update.

This can be carried out either with:

-	the BSSMAP Classmark procedure(s).

Apart from these cases there is the "odd" case where a Classmark Update can be received during an Inter-MSC Handover by MSC-B, i.e. before the MS has moved to the new channel controlled by MSC-B. This can be made with transparent transfer of BSSMAP Classmark Update.

#### 4.5.5.4	Downlink DTX-Flag

The parameter shall be stored by MSC-B to be used at internal Handover in MSC-B.

Transfer of Information:

Received by MSC-B from MSC-A in either:

If the MS has already been assigned to a TCH for speech before the Inter-MSC Handover, the DTX-flag should be sent in:

-	the Handover Request BSSMAP message;

(if the type of resource is not TCH for speech, the DTX-flag shall not be included).

If a new assignment to a TCH for speech after an Inter-MSC Handover is to be performed, this can be made with:

-	the BSSMAP Assignment procedure.

#### 4.5.5.5	Priority

The parameter shall be stored by MSC-B and is received according to below:

Transfer of Information:

Received by MSC-B from MSC-A in:

-	the Handover Request BSSMAP message.

If a change is needed after an Inter-MSC Handover with:

-	the BSSMAP Assignment procedure.

#### 4.5.5.6	MSC/BSC-Invoke Trace Information Elements

The process to be performed by MSC-B on the information elements of the MSC or BSC Invoke Trace BSSMAP messages is left for further study.

#### 4.5.5.7	LSA Identifier List

The parameter shall be stored by MSC-B and is received according to below:

Transfer of Information:

Received by MSC-B from MSC-A in:

-	the Handover Request BSSMAP message.

If a change is needed after an Inter-MSC Handover with:

-	the LSA Information BSSMAP message.

### 4.5.6	Overview of the Technical Specifications GSM interworking for the Inter-MSC Handover

╔══════════════════════════════════════════════════════════════════════════════════════════════════╗

║ PSTN/ISDN                                                                                        ║

║               ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                 ║

║               ▒       +---------------+                        ▒                                 ║

║ ==============▒====== | ============= | ===============        ▒                                 ║

║ PLMN          ▒       |               |                        ▒                                 ║

║               ▒       V               V                        ▒                                 ║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                                ▒                                 ║

║ ▒  BSS-A            MSC-A           MSC-B          BSS-B       ▒     MS                          ║

║ ▒                                                              ▒                                 ║

║ ▒                 +-------+                                    ▒ +-------+                       ║

║ ▒                 |  C M  |                 04.08 (Note)       ▒ |  C M  |                       ║

║ ▒                 +-------|<------------------------------------>+-------|                       ║

║ ▒ ░░░░░░░░░░░░░░░ |  M M  | ░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░▒  |  M M  |                       ║

║ ▒ ░+------+ 08.08 +-------| 08.08'+-------+ 08.08 +-------+  ░▒  +-------|                       ║

║ ▒ ░| R R  |<----->|  R R  |<=====>|  R R  |<----->|  R R  |  ░▒  |  R R  |                       ║

║ ▒ ░+------+       +-------+ (Note)+-------+       +-------+  ░▒  +-------+                       ║

║ ▒ ░                                                      ░▒                                  ║

║ ▒ ░                |   |    09.10    |   |                   ░▒  Note: Subset of 08.08           ║

║ ▒ ░                |   V             V   |                   ░▒        procedures as described   ║

║ ▒ ░                |+-----+ 09.02 +-----+|                   ░▒        in the TS GSM 08.08.      ║

║ ▒ ░                ||MAP/E|<----->|MAP/E||                   ░▒                                  ║

║ ▒ ░                |+-----+       +-----+|              03.09░▒                                  ║

║ ▒ ░░░░░░░░░░░░░░░░ | ░░░░░░░░░░░░░░░░░░░ | ░░░░░░░░░░░░░░░░░░░▒  Remark: The A-bis interface and ║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒          the link layer          ║

║                   +-----------------------+                               protocols being out of ║

║                   |       T C A P         |                               the scope of this      ║

║                   +-----------------------+                               Specification are not  ║

║    +------------------------------------------------------+               shown here.            ║

║    |                      S C C P                         |                                      ║

║    +------------------------------------------------------+                                      ║

║    +------------------------------------------------------+                                      ║

║    |                        M T P                         |                                      ║

║    +------------------------------------------------------+                                      ║

╚══════════════════════════════════════════════════════════════════════════════════════════════════╝

## 4.6	Inter-MSC Handover (UMTS to GSM)

The general principles of the handover procedures are given in 3GPP TS 23.009. 3GPP TS 29.010 gives the necessary information for interworking between the 3GPP TS 25.413 RANAP protocol, GSM handover procedures and the 3GPP TS 29.002 MAP protocol. The RANAP protocol is used between the RNS and the 3G-MSC.

The following three principles apply for the Inter-MSC handover UMTS to GSM:

The BSSMAP parameters required for Inter-MSC handover UMTS to GSM are generated as in GSM.

Received BSSMAP parameters, e.g. cause code or Handover command, are mapped to the appropriate RANAP parameters, e.g. cause code transparent container to source RNS.

### Basic Inter-MSC Handover

When a Mobile Station is handed over between two MSCs, the establishment of a connection between them (described in 3GPP TS 23.009) requires interworking between A-Interface and E-Interface.

The signalling at initiation, execution, completion of the Basic Inter-MSC handover procedure is shown in figures 21 to 26 with both possible positive or negative outcomes.

Additionally figure 21b shows the possible interworking when the trace related message is transparently transferred on the E-Interface at Basic Inter-MSC Handover initiation.

RNS-A         3G-MSC-A                   MSC-B

|               |                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request                  | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 21a: Signalling for Basic Inter-MSC Handover initiation (no trace related messages transferred)

RNS-A         3G-MSC-A                   MSC-B

|             (*)                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request(**)              | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

|               |                         |                   |

|               |                         |MSC INVOKE TRACE   |

|               |                         |--------------->(***)

Figure 21b: Signalling for Basic Inter-MSC Handover initiation (MSC invoke trace message transferred)

(*):	Tracing invocation has been received from VLR.

(**):	In that case, HANDOVER REQUEST and MSC INVOKE TRACE messages are included within the BSS-apdu parameter.

(***):	MSC INVOKE TRACE is forwarded to BSS-B if supported by MSC-B.

Possible Positive outcomes:

a)	successful radio resources allocation and handover number allocation (if performed):

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE HANDOVER    |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|RELOCATION COMMAND                       |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued and successful handover number allocation (if performed). Later successful radio resources allocation indication:

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|

| RELOCATION COMMAND   SIGNALLING request |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 22: Signalling for Basic Inter-MSC Handover execution (Positive outcomes)

Possible Negative outcomes:

c)	user error detected, or handover number allocation unsuccessful (if performed), or component rejection or dialogue abortion performed by MSC-B:

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|               |MAP PREPARE HANDOVER response                |

|               |negative result, MAP CLOSE                   |

|               |<------------------------|                   |

|               |MAP U/P-ABORT            |                   |

|               |                         |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

|               |                         |                   |

d)	radio resources allocation failure:

RNS-A        3G-MSC-A                    MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

|               |                         |                   |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

|               | SIGNALLING request      |                   |

|               |                         |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

f)	unsuccessful handover execution (Reversion to the old radio resources):

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|RELOCATION     |                         |                   |

|-------------->|                         |                   |

|CANCEL         |                         |                   |

|               |MAP U -ABORT             |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|RELOCATION     |                         |                   |

|<--------------|                         |                   |

|CANCEL ACK     |                         |                   |

Figure 23: Signalling for Basic Inter-MSC Handover execution (Negative outcomes)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or RANAP procedure.

RNS-A         3G-MSC-A                    MSC-B              BSS-B

|               |                         |                   |

|               |                         |HANDOVER COMPLETE  |

|               |                         |<------------------|

|               |MAP SEND END SIGNAL request                  |

|               |<------------------------|                   |

|IU RELEASE COMMAND                       |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 24: Signalling for Basic Inter-MSC Handover completion

Positive outcome:

RNS-A         3G-MSC-A                   MSC-B              BSS-B

|               |                         |                   |

|               |MAP SEND END SIGNAL      |                   |

|               |------------------------>|                   |

|               | response                |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         | (Note 2)          |

Figure 25: Signalling for Basic Inter-MSC Handover completion (Positive outcome)

Negative outcome:

RNS-A         3G-MSC-A                    MSC-B              BSS-B

|               |                         |                   |

|               | MAP U/P -ABORT          |                   |

|               |------------------------>|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |                   |

Figure 26: Signalling for Basic Inter-MSC Handover completion (Negative outcome)

NOTE 2:	From interworking between MAP and BSSMAP point of view, when the call is released.

The handover procedure is normally triggered by RNS-A by sending a RELOCATION REQUIRED message on Iu-Interface to 3G-MSC-A. The invocation of the Basic Inter-MSC handover procedure is performed and controlled by 3G-MSC-A. The sending of the MAP Prepare-Handover request to MSC-B is triggered in 3G-MSC-A upon receipt of the RELOCATION REQUIRED message. For compatibility reason, the cell identity of the cell where the call is to be handed over in MSC-B area, provided in the RELOCATION REQUIRED message, is mapped into targetCellId MAP parameter and the HANDOVER REQUEST message is encapsulated in the bss-APDU MAP parameter of the Prepare-Handover MAP request. MSC-B can invoke another operation towards the VLR-B (allocation of the handover number described in 3GPP TS 29.002).

Additionally, if tracing activity has been invoked, the trace related message can be transferred on the E-Interface encapsulated in the bss-APDU MAP parameter of the Prepare-Handover Request. If transferred, one complete trace related message at a time shall be included in the bss-APDU MAP parameter after the HANDOVER REQUEST message.

The interworking between Prepare Handover and RELOCATION REQUIRED is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION REQUIRED MAP PREPARE HANDOVER request|

message |                                                 |

|                            -ho-NumberNotRequired|   1

|  RANAP information        -targetCellId        |

|         elements           -bss-APDU(           |   2

|                            HANDOVER REQUEST,    |

| |

| MSC INVOKE TRACE)    |

--------┼-------------------------------------------------┼-----

Positive| RELOCATION CMD     MAP PREPARE HANDOVER response|

result  |                                                 |   3

|                             -handover number    |

|                             -bss-APDU(          |

|                              QUEUING INDICATION |

|                              or HANDOVER REQUEST|

|                              ACKNOWLEDGE)       |

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| RELOCATION PREP FAILURE     MAP PREPARE HANDOVER|   4

result  |                                                 |

| Relocation failure in            System Failure     |

| target RNC/CN or target system|

|”            No Handover Number |

|                                 available       |

|”            UnexpectedDataValue|

|”            Data Missing       |

|                                                 |

|”            MAP CLOSE          |

|”            MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The BSSMAP information elements are already stored in 3G-MSC.

The ho-NumberNotRequired parameter is included by 3G-MSC-A, when 3G-MSC-A decides not to use any circuit connection with MSC-B. No handover number shall be present in the positive result. Any negative response from MSC-B shall not be due to handover number allocation problem.

NOTE 2:	The process performed on the RANAP information elements received in the RELOCATION REQUIRED message is described in the 3GPP TS 25.413.

NOTE 3:	The response to the Prepare-Handover request can include in its bss-APDU parameter, identifying the GSM 08.06 protocol, either a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE.

In the first case, 3G-MSC-A shall wait for the radio resources allocation response from MSC-B, transmitted to 3G-MSC-A as described in subclause 4.5.4.

In the second case, the positive result triggers in 3G-MSC-A the sending on Iu-Interface of the RELOCATION CMD.

In the third case, the positive result triggers in 3G-MSC-A.

NOTE 4:	The possible sending of the RELOCATION PREP FAILURE message is described in the 3G 25.413.

(The possible sending of the RELOCATION PREP FAILURE message upon receipt of the HANDOVER FAILURE is out of the scope of the 3GPP TS 29.010 and lies in the 3GPP TS 25.413).

The interworking between Send End Signal and HANDOVER COMPLETE in MSC-B is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER COMPLETE   MAP SEND END SIGNAL request |

message |                                                 |

|                               -bss-APDU(        |

|                               HANDOVER COMPLETE)|

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| CLEAR COMMAND       MAP SEND END SIGNAL response|

result  |  -Call Control release                          |  1

--------┼-------------------------------------------------┼-----

Negative| CLEAR COMMAND                                   |

result  |  -Call Control release        MAP CLOSE         |  2

|  -Call Control release        MAP U/P -ABORT    |

|                                                 |

NOTE 1:	The positive empty result triggers the clearing of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B. If a circuit connection is used between 3G_MSC-A and MSC-B, the 'Call Control release' clearing cause shall only be given to BSS-B when MSC-B has received a clearing indication on its circuit connection with 3G_MSC-A.

NOTE 2:	The abortion of the dialogue or the rejection of the component triggers in MSC-B the clearing of its circuit connection with 3G_MSC-A, if any, of the Radio Resources on the A-Interface and the release of the SCCP connection between MSC-B and BSS-B.

The interworking between Send End Signal and  IU RELEASE COMMAND in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   29.002                          25.413        |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP SEND END SIGNAL          IU RELEASE COMMAND |

message | response                                        |

|          -bss-APDU(           -        |

|          HANDOVER COMPLETE)        Successful Relocation   |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

The interworking between RELOCATION CANCEL in case of reversion to old channel of the UE and User Abort in 3G-MSC-A is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION CANCEL               MAP U -ABORT    |

message |                                                 |

|  -Relocation cancelled                                      |

--------┼-------------------------------------------------┼-----

Positive| RELOCATION CANCEL ACKNOWLEDGEMENT               |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

### Subsequent Inter-MSC Handover from  3G-MSC-B back to MSC-A

When a Mobile Station is being handed over back to MSC-A, the procedure (described in TS 23.009) requires interworking between A-Interface, Iu-interface and E-Interface.

The signalling at initiation, execution and completion of the Subsequent Inter-MSC handover procedure is shown in figures 27 to 31.

RNS-A         3G-MSC-B                   MSC-A

|               |                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE SUBSEQUENT   |

|REQUIRED       |------------------------>|

|               |HANDOVER request         |

|               |                         |                BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |------------------>|

Figure 27: Signalling for Subsequent Inter-MSC Handover back to MSC-A initiation

Possible Positive outcomes:

a)	successful radio resources allocation:

RNS-A          3G-MSC-B                  MSC-A              BSS-B

|               |                         |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|RELOCATION CMD |                         |                   |

|<--------------|                         |                   |

b)	radio resources allocation queued. Later successful radio resources allocation indication:

RNS-A         3G-MSC-B                   MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|               |                         |HANDOVER REQUEST   |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP FORWARD ACCESS      |                   |

|               |<------------------------|                   |

|               | SIGNALLING request      |                   |

|RELOCATION CMD |                         |                   |

|<--------------|                         |                   |

Figure 28: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution (Positive outcome)

Possible Negative outcomes:

c)	user error detected, or component rejection or dialogue abortion performed by MSC-A:

RNS-A         3G-MSC-B                  MSC-A              BSS-B

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|RELOCATION PREP| response negative result|                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

|               |                         |                   |

d)	component rejection or dialogue abortion performed by MSC-A:

RNS-A         3G-MSC-B                  MSC-A              BSS-B

|               |MAP CLOSE, MAP U/P ABORT |                   |

|               |<------------------------|                   |

|IU RELEASE     |                         |                   |

|<--------------|                         |                   |

|   COMMAND     |                         |                   |

e)	radio resources allocation failure:

RNS-A         3G-MSC-B                  MSC-A              BSS-B

|               |                         | HANDOVER FAILURE  |

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|RELOCATION PREP|    HANDOVER response    |                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

f)	radio resources allocation queued. Later unsuccessful radio resources allocation:

RNS-A         3G-MSC-B                  MSC-A              BSS-B

|               |                         |QUEUING INDICATION |

|               |                         |<------------------|

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               |    HANDOVER response    |                   |

|               |                         |HANDOVER FAILURE   |

|               |                         |<------------------|

|               | MAP FORWARD ACCESS      |                   |

|               |<------------------------|                   |

|RELOCATION PREP|    SIGNALLING request   |                   |

|<--------------|                         |                   |

|FAILURE(Note 1)|                         |                   |

Figure 29: Signalling for Subsequent Inter-MSC Handover back to MSC-A execution
(Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

BSS-B         MSC-A                   3G-MSC-B            RNS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |Iu RELEASE COMMAND |

|               |                         |------------------>|

Figure 30: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion
(Successful completion of the procedure)

NOTE:	Positive outcome case shown in figure 9.

BSS-B         MSC-A                   3G-MSC-B             RNS-A

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |                   |

|               |MAP U/P -ABORT           |                   |

|               |<------------------------|                   |

|               |                         |Iu RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |(Note 1)           |

Figure 31: Signalling for Subsequent Inter-MSC Handover back to MSC-A completion (Unsuccessful completion of the procedure)

NOTE 1:	Abnormal end of the procedure which triggers the clearing of all resources in 3G-MSC-B.

The interworking between Prepare Subsequent Handover and RELOCATION REQUIRED is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward |REL. REQUIRED     MAP PREPARE SUBSEQUENT HANDOVER|

message |                              request            |

|                                                 |

|                              -target MSC number |

|                              -targetCellId      |

|                              -bss-APDU(         |

|                               HANDOVER REQUEST) |

|                                                 |

|                                                 |

| RANAP information            BSSMAP information |

|        elements:                elements:       |

|                                                 |

| MS Classmark 2                CM2               |

| Source Id                     Cell Id (serving) |

| Target Id                     Cell Id (target)  |

| Cause                         Cause             |1

| MS Classmark 3                CM3               |

|                                                 |

|                           info stored/generated |

|                           in/by 3G-MSC-B:       |

|                               Message Type      |

|                               Channel Type      |

|                               Speech version    |

|                               Priority          |

|                               Interference Band |

|                               to be used        |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|RELOCATION CMD.   MAP PREPARE SUBSEQUENT HANDOVER|

result  |                             response            |  2

|                             -bss-APDU(          |

|                              QUEUING INDICATION |

|                              or HANDOVER REQUEST|

|                              ACKNOWLEDGE or     |

|                              HANDOVER FAILURE)  |

|                                                 |

| RANAP information            BSSMAP information |

|        elements:                elements:       |

| L3 information               L3 information     |

|                                                 |

|                                                 |

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| REL. PREP. FAILURE        MAP PREPARE SUBSEQUENT|  3

result  |                           HANDOVER response     |

| Relocation Failure in Target CN/RNC or Target System Unknown MSC                                               |

| Relocation Failure in Target CN/RNC or Target System Subsequent Handover                                       |

|                                         Failure |

| Relocation Failure in Target CN/RNC or Target System

|                                                 |

UnexpectedDataValue                                       |

| Relocation Failure in Target CN/RNC or Target System Data Missing                                              |

|                                                 |

| Iu RELEASE COMMAND                              |

|                                                 |

| Relocation Cancelled MAP CLOSE                  |

| Relocation Cancelled MAP U/P -ABORT             |

|                                                 |

NOTE 1:	The mapping of cause code values between BSSMAP and RANAP is FFS.

NOTE 2:	The response to the Prepare-Subsequent-Handover request can include in its bss-APDU parameter, identifying the GSM 08.06 protocol, a BSSMAP QUEUING INDICATION, or a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, 3G-MSC-B shall wait for the radio resources allocation response from MSC-A, transmitted to 3G-MSC-B as described in subclause 4.5.4.

In the second case, the positive result triggers in 3GMSC-B the sending on Iu-Interface of the RELOCATION COMMAND.

In the third case, the positive result triggers in 3G-MSC-B the sending of the RELOCATION PREPARATION FAILURE.

NOTE 3:	The possible sending of the RELOCATION PREPARATION FAILURE message is described in  3GPP TS 25.413.

The interworking between Send End Signal Result and HANDOVER COMPLETE in MSC-A is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER COMPLETE    MAP SEND END SIGNAL        |

message |                               response          |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                          MAP U/P -ABORT         |  1

|                                                 |

NOTE:	The abortion of the dialogue ends the handover procedure with 3G-MSC-B.

### Subsequent Inter-MSC Handover to third MSC

When a Mobile Station is being handed over to a third MSC, the procedure (described in 3GPP TS 23.009) does require one specific interworking case in MSC-A between E-Interface from 3G-MSC-B and E-Interface from MSC-B' other than the combination of the ones described in subclauses 4.6.1 and 4.6.2.

RNS-A      3G-MSC-B               MSC-A            MSC-B'

|            |                      |                |

|RELOCATION  |                      |                |

|----------->|MAP PREPARE SUBSEQUENT|                |

|REQUIRED    |--------------------->|                |

|            |HANDOVER request      |MAP PREPARE     |

|            |                      |--------------->|

|            |                      |HANDOVER request|

|            |                      |                |+-------+

|            |                      |                ||Possib.|

|            |                      |                ||Alloc. |

|            |                      |                ||of ho. |

|            |                      |                ||number |

|            |                      |                || VLR-B |

|            |                      |                |+-------+

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |-------->|

|            |                      |                |REQUEST  |

|            |                      |                |

Figure 32: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') initiation

Possible Positive outcomes:

a)	successful radio resources allocation:

RNS-A    3G-MSC-B                 MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|    response    |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

|            |                      |                |         |

b)	radio resources allocation queued and successful handover number allocation, if performed. Later successful radio resources allocation indication:

RNS-A    3G-MSC-B                 MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|   response     |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |MAP FORWARD ACCESS    |SIGNALLING request        |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 33: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution
(Positive outcome)

Possible Negative outcomes:

c)	user error detected, or component rejection or dialogue abortion performed by MSC-B':

RNS-A      3G-MSC-B               MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |response negative result  |

|            |                      |MAP CLOSE       |         |

|            |                      |<---------------|         |

|            |                      |MAP U/P -ABORT  |         |

|            |MAP PREPARE SUBSEQUENT|                |         |

|            |<---------------------|                |         |

|            |HANDOVER response negative             |         |

| RELOCATION |result                |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

| (Note 1)   |                      |                |         |

d)	radio resources allocation failure:

RNS-A      3G-MSC-B               MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

| (Note 1)   |                      |                |         |

e)	radio resources allocation queued and successful handover number allocation (if performed). Later unsuccessful radio resources allocation:

RNS-A      3G-MSC-B               MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                | QUEUING |

|            |                      |                |<--------|

|            |                      |                |INDICAT. |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PROCESS ACCESS        |

|            |                      |<---------------|         |

|            |                      |SIGNALLING request        |

|            |MAP FORWARD ACCESS    |                |         |

|            |<---------------------|                |         |

|            |SIGNALLING request    |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

| (Note 1)   |                      |                |         |

Figure 34: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') execution (Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

Positive outcome:

RNS-A    3G-MSC-B                 MSC-A            MSC-B'

|            |                      |                |

|            |                      |                |    BSS-B'

|            |                      |                |         |

|            |                      |                |HANDOVER |

|            |                      |                |<--------|

|            |                      |                |COMPLETE |

|            |                      |                |         |

|            |                      |MAP SEND END SIGNAL       |

|            |                      |<---------------|         |

|            | MAP SEND END SIGNAL  |                |         |

|            |<---------------------|                |         |

|            | response             |                |         |

| IU-RELEASE |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 35: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Successful completion of the procedure)

Negative outcome:

RNS-A      3G-MSC-B               MSC-A            MSC-B'

|            |                      |                |

|RELOCATION  |                      |                |    BSS-B'

|----------->|                      |                |         |

|CANCEL      |MAP PROCESS ACCESS    |                |         |

|            |--------------------->|                |         |

|            |SIGNALLING request (Note 1)            |         |

|            |                      |                |         |

|            |                      |MAP U -ABORT    |         |

|            |                      |--------------->|         |

|            |                      |                |CLEAR    |

|            |                      |                |-------->|

|            |                      |                |COMMAND  |

|RELOCATION  |                      |                |         |

|<-----------|                      |                |         |

|CANCEL ACK  |                      |                |         |

|            |                      |                |         |

|            |                      |                |         |

|            |                      |                |         |

|            |                      |                |         |

Figure 36: Signalling for Subsequent Inter-MSC Handover to third MSC (MSC-B') completion (Unsuccessful completion of the procedure)

NOTE 1:	Specific interworking case detailed below.

The specific interworking case in MSC-A compared to the subclauses 4.5.1 and 4.5.2 occurs between HANDOVER FAILURE encapsulated in a Process Access Signalling from 3G-MSC-B and the abortion of the dialogue with MSC-B' in the case of a reversion to old channel of the MS:

----------------------------------------------------------------

|   29.002                          29.002        |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PROCESS-SIGNALLING                          |

message |         request                                 |

|                                                 |

|         -bss-APDU(               MAP U -ABORT   |  1

|         HANDOVER FAILURE)                       |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                  MAP U/P -ABORT |  2

|                                                 |

NOTE 1:	The abortion of the dialogue triggers in MSC-B' the clearing of the circuit connection with MSC-A, if any, and of the Resources between MSC-B' and BSS-B'.The abortion of the dialogue ends the handover procedure with MSC-B'.

NOTE 2:	The abortion of the dialogue ends the handover procedure with 3G-MSC-B.

### BSSAP Messages transfer on E-Interface

The handling is described in chapter 4.5.4.

### Cause Code Mapping

When a Mobile Station is handed over between UMTS and GSM, a mapping of the cause codes used in the RANAP and the BSSMAP protocols is needed. The mapping described here is applicable to the BSSMAP protocol even when used inside MAP in the E-interface.

The mapping between the cause codes received in RANAP Relocation Required and the cause codes sent in BSSMAP Handover Request is as follows:

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

RELOCATION REQUIRED            HANDOVER REQUEST        |

|

-Time critical relocation      -Better cell           |

-Resource optimisation                                |  1

relocation                                           |

-Relocation desirable for      -Better cell           |

radio reasons                                        |

-Directed retry                -Directed retry        |

-Any other value               -Better cell           |

NOTE 1: Cause code not used at inter-system handover.

The mapping between the cause codes received in RANAP Relocation Cancel and the cause codes sent in BSSMAP Clear Command is as follows:

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

RELOCATION CANCEL              CLEAR COMMAND           |

|

-Trelocprepexpiry              -Radio interface       |

failure, reversion to |

old channel           |

-Interaction with other        -Radio interface       |

procedure                      failure, reversion to |

old channel           |

-Any other value               -Radio interface       |

failure, reversion to |

old channel           |

The mapping between the cause codes received in BSSMAP Handover Failure and the cause codes sent in RANAP Relocation Preparation Failure is as follows:

----------------------------------------------------------------

08.08                         25.413              |Notes

-------------------------------------------------------┼--------

HANDOVER FAILURE               RELOCATION PREP. FAILURE|

|

-Ciphering algorithm not       -Requested ciphering   |

supported                      and/or integrity      |

protection is not     |

supported             |

-Circuit pool mismatch                                |	 1

-Equipment failure             -Relocation failure in |

Target CN/RNC or      |

target system         |

-Invalid message contents      - Abstract Syntax Error|

-No radio resource available   -Relocation failure in |

Target CN/RNC or      |

target system         |

-O and M intervention          -O and M intervention  |

-Radio interface failure,                             |	 2

reversion to old channel                             |

-Radio interface message       -Relocation failure in |

failure                        Target CN/RNC or      |

target system         |

-Requested speech version      -Relocation failure in |

unavailable                    Target CN/RNC or      |

target system         |

-Requested terrestrial         -Relocation failure in |

resource unavailable           Target CN/RNC or      |

target system         |

-Requested transcoding/rate    -Relocation failure in |

adaption unavailable           Target CN/RNC or      |

target system         |

-Switch circuit pool                                  |	 1

-Terrestrial circuit already   -Relocation failure in |

allocated                      Target CN/RNC or      |

target system         |

-Any other value               -Relocation failure in |

Target CN/RNC or      |

target system         |

NOTE 1: Cause code not used at inter-system handover.

NOTE 2: Cause code not applicable to this traffic case.

## 4.7	Inter-MSC Handover (GSM to UMTS)

The general principles of the handover procedures are given in 3GPP TS 23.009. 3GPP TS 29.010 gives the necessary information for interworking between the 3GPP TS 25.413 RANAP protocol, GSM handover procedures and the 3GPP TS 29.002 MAP protocol. The RANAP protocol is used between the RNS and the 3G_MSC.

The following four principles apply for the Inter-MSC handover GSM to UMTS:

The BSSMAP parameters required for Inter-MSC handover GSM to UMTS are generated as in GSM.

Received RANAP parameters, e.g. cause code or transparent container, are mapped to the appropriate BSSMAP parameters, e.g. cause code or Handover command.

The RANAP parameters required for Inter-MSC handover GSM to UMTS are generated from received or stored GSM parameters.

### 4.7.1	Basic Inter-MSC Handover

When a Mobile Station is handed over between two MSCs, the establishment of a connection between them (described in 3GPP TS 23.009) requires interworking between A-Interface, Iu-Interface and E-Interface.

The signalling at initiation, execution and completion of the Basic Inter-MSC handover procedure is shown in figures 37 to 42 with both possible positive or negative outcomes.

Additionally figure 37b shows the possible interworking when the trace related message is transparently transferred on the E-Interface at Basic Inter-MSC Handover initiation.

BSS-A         MSC-A                   3G-MSC-B

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request                  | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

Figure 37a: Signalling for Basic Inter-MSC Handover initiation (no trace related messages transferred)

BSS-A         MSC-A                   3G-MSC-B

|             (*)                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request  (**)            | |Possible Alloc. |

|               |                         | |of a handover   |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

|               |                         |                   |

|               |                         |CN INVOKE TRACE    |

|               |                         |--------------->(***)

Figure 37b: Signalling for Basic Inter-MSC Handover initiation (CN invoke trace message transferred)

(*):	Tracing invocation has been received from VLR.

(**):	In that case, HANDOVER REQUEST and MSC INVOKE TRACE messages are included within the AN-apdu parameter.

(***):	CN INVOKE TRACE is forwarded to RNS-B if supported by 3G_MSC-B.

Possible Positive outcomes: successful radio resources allocation and handover number allocation (if performed):

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               |                         |LOCATION REPORTING |

|               |                         |------------------>|

|               |                         |CONTROL            |

|               | MAP PREPARE HANDOVER    |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

Figure 38: Signalling for Basic Inter-MSC Handover execution (Positive outcome)

Possible Negative outcomes:

a)	user error detected, or handover number allocation unsuccessful (if performed), or component rejection or dialogue abortion performed by 3G_MSC-B:

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |MAP PREPARE HANDOVER response                |

|               |negative result, MAP CLOSE                   |

|               |<------------------------|                   |

|               |MAP U/P-ABORT            |                   |

|               |                         |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

b)	radio resources allocation failure:

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |                         |RELOCATION FAILURE |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|HANDOVER REQUIRED                        |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

c)	unsuccessful handover execution (Reversion to the old radio resources):

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|HANDOVER       |                         |                   |

|-------------->|                         |                   |

|FAILURE        |                         |                   |

|               |MAP U -ABORT             |                   |

|               |------------------------>|                   |

|               |                         |IU RELEASE COMMAND |

|               |                         |------------------>|

Figure 39: Signalling for Basic Inter-MSC Handover execution (Negative outcomes)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or RANAP procedure.

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |                         |RELOCATION COMPLETE|

|               |                         |<------------------|

|               |MAP SEND END SIGNAL request                  |

|               |<------------------------|                   |

|CLEAR COMMAND  |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 40: Signalling for Basic Inter-MSC Handover completion

Positive outcome:

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |MAP SEND END SIGNAL      |                   |

|               |------------------------>|                   |

|               | response                |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         | (Note 2)          |

Figure 41: Signalling for Basic Inter-MSC Handover completion (Positive outcome)

Negative outcome:

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               | MAP U/P -ABORT          |                   |

|               |------------------------>|                   |

|               |                         |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |                   |

Figure 42: Signalling for Basic Inter-MSC Handover completion (Negative outcome)

NOTE 2:	From interworking between MAP and RANAP point of view, when the call is released.

BSS-A         MSC-A                   3G-MSC-B              RNS-B

|               |                         |                   |

|               |                         |LOCATION REPORT    |

|               |                         |<------------------|

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

|               |SIGNALLING               |                   |

Figure 42a: Signalling for updating of anchor MSC after change of location in RNS

The handover procedure is normally triggered by BSS-A by sending a HANDOVER REQUIRED message on A-Interface to MSC-A. The invocation of the Basic Inter-MSC handover procedure is performed and controlled by MSC-A. The sending of the MAP Prepare-Handover request to 3G_MSC-B is triggered in MSC-A upon receipt of the HANDOVER REQUIRED message. The identity of the target RNC where the call is to be handed over in 3G_MSC-B area, provided in the HANDOVER REQUIRED message in the information element Cell Identifier List (Preferred), is mapped to the target RNC Id MAP parameter and the HANDOVER REQUEST message is encapsulated in the an-APDU MAP parameter of the Prepare-Handover MAP request. 3G_MSC-B can invoke another operation towards the VLR-B (allocation of the handover number described in 3GPP TS 29.002).

Additionally, if tracing activity has been invoked, the trace related message can be transferred on the E-Interface encapsulated in the an-APDU MAP parameter of the Prepare-Handover Request. If transferred, one complete trace related message at a time shall be included in the an-APDU MAP parameter after the HANDOVER REQUEST message. Note: UMTS supports only CN initiated tracing.

The interworking between Prepare Handover and HANDOVER REQUIRED is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER REQUIRED   MAP PREPARE HANDOVER request|

message |                                                 |

|                            -ho-NumberNotRequired| 1

|  BSSMAP information        -target RNC Id       |

|         elements           -IMSI                |

|                            -Integrity protection|	2

|                                info             |

|                            -Encryption info     |

|                            -an-APDU(            | 3

|                            HANDOVER REQUEST,    |

|                            MSC INVOKE TRACE)    | 4

--------┼-------------------------------------------------┼-----

Positive|                    MAP PREPARE HANDOVER response|

result  |                                                 | 5

|                             -handover number    |

|                             -an-APDU(           |

|                              HANDOVER REQUEST   |

|                              ACKNOWLEDGE or     |

|                              HANDOVER FAILURE)  |

--------┼-------------------------------------------------┼-----

Negative| HANDOVER REQUIRED REJECT    MAP PREPARE HANDOVER| 6

result  |                                                 |

| equipment failure            System Failure     |

| equipment failure            No Handover Number |

|                                 available       |

| equipment failure            UnexpectedDataValue|

| equipment failure            Data Missing       |

|                                                 |

| equipment failure            MAP CLOSE          |

| equipment failure            MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The ho-NumberNotRequired parameter is included by MSC-A, when MSC-A decides not to use any circuit connection with 3G_MSC-B. No handover number shall be present in the positive result. Any negative response from 3G_MSC-B shall not be due to handover number allocation problem.

NOTE 2:	Integrity protection information, encryption information and IMSI parameters are included by MSC-A, only when the MSC-A uses 29.002 as per release 99. These IEs are not included if the MSC-A is R98 or earlier.

NOTE 3:	The process performed on the BSSMAP information elements received in the HANDOVER REQUIRED message is described in the GSM Recommendation 08.08.

NOTE 4:	The process performed on the BSSMAP information elements received in the MSC INVOKE TRACE message is described in subclause 4.5.5.6.

NOTE 5:	The response to the Prepare-Handover request can include in its an-APDU parameter, identifying the GSM 08.06 protocol, either a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, the positive result triggers in MSC-A the sending on A-Interface of the HANDOVER COMMAND.

In the second case, the positive result triggers in MSC-A optionally the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of 3GPP TS 29.010 and lies in GSM 08.08).

NOTE 6:	The possible sending of the HANDOVER REQUIRED REJECT message is described in GSM 08.08.

The interworking between Prepare Handover and RELOCATION REQUEST in 3G_MSC-B is as follows:

----------------------------------------------------------------

|   29.002                         25.413         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PREPARE HANDOVER    RELOCATION REQUEST      |

message | request                                         |

| -ho-NumberNotRequired                           |

| -target RNC Id                                  |

| -IMSI                                           |

| -Integrity protection info                      |	1

| -Encryption info                                |

| -an-APDU(                                       |

| HANDOVER REQUEST,                               |

| MSC INVOKE TRACE)                               |

|                                                 |

| BSSMAP information      RANAP information       |

|      elements:              elements:           |

|                                                 |

|   Channel Type            RAB parameters        |

|   Cause                   Cause                 |

|   sRNC to tRNC container  sRNC to tRNC container|

|                                                 |

|                         info stored/generated   |

|                         in/by 3G_MSC-B:         |

|                           CN domain indicator   |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP PREPARE HANDOVER    RELOCATION REQUEST ACK  |

result  | response                                        |

| -an-APDU(                                       |

|  HANDOVER REQUEST ACK)                          |

|                                                 |

| BSSMAP information      RANAP information       |

|      elements:              elements:           |

|                                                 |

|   Layer 3 info            tRNC to sRNC container|

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| MAP PREPARE HANDOVER    RELOCATION FAILURE      |

result  | response                                        |

| -an-APDU(                                       |

|  HANDOVER FAILURE)                              |

|                                                |

NOTE 1:	Integrity protection information, encryption information and IMSI parameters are included by MSC-A, only when the MSC-A uses 29.002 as per release 99. These IEs are not included if the MSC-A is R98 or earlier.

The interworking between Send End Signal and RELOCATION COMPLETE in 3G_MSC-B is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION COMPLETE MAP SEND END SIGNAL request |

message |                                                 |

|                               -an-APDU(         |

|                               HANDOVER COMPLETE)|

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| IU RELEASE COMMAND  MAP SEND END SIGNAL response|

result  |  -Normal release                                |  1

--------┼-------------------------------------------------┼-----

Negative| IU RELEASE COMMAND                              |

result  |  -Normal release                    MAP CLOSE   |  2

|  -Normal release                  MAP U/P -ABORT|

|                                                 |

NOTE 1:	The positive empty result triggers the clearing of the Radio Resources on the Iu-Interface and the release of the SCCP connection between 3G_MSC-B and RNS-B. If a circuit connection is used between MSC-A and 3G_MSC-B, the 'Normal release' clearing cause shall only be given to RNS-B when 3G_MSC-B has received a clearing indication on its circuit connection with MSC-A.

NOTE 2:	The abortion of the dialogue or the rejection of the component triggers in 3G_MSC-B the clearing of its circuit connection with MSC-A, if any, of the Radio Resources on the Iu-Interface and the release of the SCCP connection between 3G_MSC-B and RNS-B.

The interworking between Send End Signal and CLEAR COMMAND in MSC-A is as follows:

----------------------------------------------------------------

|   29.002                         08.08          |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP SEND END SIGNAL          CLEAR COMMAND      |

message | request                                         |

|          -an-APDU(              - Handover      |

|          HANDOVER COMPLETE)        Successful   |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

The interworking between HANDOVER FAILURE in case of reversion to old channel of the MS and User Abort in MSC-A is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER FAILURE                MAP U -ABORT    |

message |                                                 |

|  - Reversion to old                             |

|    channel                                      |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

### 4.7.2	Subsequent Inter-MSC Handover from MSC-B back to 3G_MSC-A

When a Mobile Station is being handed over back to 3G_MSC-A, the procedure (described in 3GPP TS 23.009) requires interworking between A-Interface, Iu-Interface and E-Interface.

The signalling at initiation, execution and completion of the Subsequent Inter-MSC handover procedure is shown in figures 43 to 47.

BSS-A         MSC-B                     3G-MSC-A

|               |                         |

|HANDOVER       |                         |

|-------------->|MAP PREPARE SUBSEQUENT   |

|REQUIRED       |------------------------>|

|               |HANDOVER request         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

Figure 43: Signalling for Subsequent Inter-MSC Handover back to 3G_MSC-A initiation

Possible Positive outcomes: successful radio resources allocation:

BSS-A         MSC-B                     3G-MSC-A           RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|HANDOVER COMMAND                         |                   |

|<--------------|                         |                   |

Figure 44: Signalling for Subsequent Inter-MSC Handover back to 3G_MSC-A execution (Positive outcome)

Possible Negative outcomes:

a)	user error detected, or component rejection or dialogue abortion performed by 3G_MSC-A:

BSS-A         MSC-B                     3G-MSC-A           RNS-B

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|HANDOVER REQUIRED  response negative result                  |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

|               |                         |                   |

b)	component rejection or dialogue abortion performed by 3G_MSC-A:

BSS-A         MSC-B                     3G-MSC-A           RNS-B

|               |MAP CLOSE, MAP U/P ABORT |                   |

|               |<------------------------|                   |

|CLEAR COMMAND  |                         |                   |

|<--------------|                         |                   |

|               |                         |                   |

c)	radio resources allocation failure:

BSS-A         MSC-B                     3G-MSC-A           RNS-B

|               |                         | RELOCATION FAILURE|

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|HANDOVER REQUIRED   HANDOVER response    |                   |

|<--------------|                         |                   |

|REJECT (Note 1)|                         |                   |

d)	unsuccessful relocation execution (reversion to the old radio resources):

BSS-A          MSC-B                  3G-MSC-A             RNS-B

|HANDOVER       |                         |                   |

|-------------->|                         |                   |

|FAILURE        |MAP PROCESS ACCESS       |                   |

|               |------------------------>|                   |

|               |   SIGNALLING request    |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |                   |

Figure 45: Signalling for Subsequent Inter-MSC Handover back to 3G_MSC-A execution
(Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

RNS-B         3G-MSC-A                  MSC-B              BSS-A

|               |                         |                   |

|RELOCATION     |                         |                   |

|-------------->|MAP SEND END SIGNAL      |                   |

|COMPLETE       |------------------------>|                   |

|               | response                |                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

Figure 46: Signalling for Subsequent Inter-MSC Handover back to 3G_MSC-A completion
(Successful completion of the procedure)

NOTE:	Positive outcome case shown in figure 41.

RNS-B         3G-MSC-A                  MSC-B              BSS-A

|               |                         |                   |

|               |MAP U/P -ABORT           |                   |

|               |<------------------------|                   |

|               |                         |CLEAR COMMAND      |

|               |                         |------------------>|

|               |                         |(Note 1)           |

Figure 47: Signalling for Subsequent Inter-MSC Handover back to 3G_MSC-A completion (Unsuccessful completion of the procedure)

NOTE 1:	Abnormal end of the procedure that triggers the clearing of all resources in MSC-B.

The interworking between Prepare Subsequent Handover and HANDOVER REQUIRED is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward |HANDOVER REQUIRED MAP PREPARE SUBSEQUENT HANDOVER|

message |                              request            |  1

|                                                 |

|                              -target MSC number |

| BSSMAP information           -target RNC Id     |

|        elements              -an-APDU(          |

|                               HANDOVER REQUEST) |

--------┼-------------------------------------------------┼-----

Positive|HANDOVER REQUIRED MAP PREPARE SUBSEQUENT HANDOVER|

result  |                             response            |  2

|                             -an-APDU(           |

|                              HANDOVER REQUEST   |

|                              ACKNOWLEDGE or     |

|                              HANDOVER FAILURE)  |

--------┼-------------------------------------------------┼-----

Negative| HANDOVER REQUIRED REJECT  MAP PREPARE SUBSEQUENT|  3

result  |                           HANDOVER response     |

| equipment failure            Unknown MSC        |

| equipment failure            Subsequent Handover|

|                                         Failure |

| equipment failure            UnexpectedDataValue|

| equipment failure            Data Missing       |

|                                                 |

| CLEAR COMMAND                                   |

|                                                 |

| equipment failure            MAP CLOSE          |

| equipment failure            MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The processing performed on the BSSMAP information elements received in the HANDOVER REQUIRED message is out of the scope of the present document. The target MSC number is provided to 3G_MSC-A by MSC-B based on the information received from RNS-B.

NOTE 2:	The response to the Prepare-Subsequent-Handover request can include in its an-APDU parameter, identifying the GSM 08.06 protocol, either a BSSMAP HANDOVER REQUEST ACKNOWLEDGE or a BSSMAP HANDOVER FAILURE.

In the first case, the positive result triggers in MSC-B the sending on A-Interface of the HANDOVER COMMAND.

In the second case, the positive result triggers in MSC-B optionally the sending of the HANDOVER REQUIRED REJECT.

(The possible sending of the HANDOVER REQUIRED REJECT message upon receipt of the HANDOVER FAILURE is out of the scope of 3GPP TS 29.010 and lies in GSM 08.08).

NOTE 3:	The possible sending of the HANDOVER REQUIRED REJECT message is described in GSM 08.08.

The interworking between Prepare Subsequent Handover and RELOCATION REQUEST in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   29.002                         25.413         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PREPARE SUB HANDOVER   RELOCATION REQUEST   |

message | request                                         |

| -ho-NumberNotRequired                           |

| -target RNC ID                                  |

| -an-APDU(                                       |

| HANDOVER REQUEST,                               |

| MSC INVOKE TRACE)                               |

|                                                 |

| BSSMAP information      RANAP information       |

|      elements:              elements:           |

|                                                 |

|   Cause                   Cause                 |

|   sRNC to tRNC container  sRNC to tRNC container|

|                                                 |

|                         info stored/generated   |

|                         in/by 3G_MSC-A:         |

|                           CN domain indicator   |

|                           RAB parameters        |

|                           Permanent NAS UE id   |

|                           Encryption info       |

|                           Integrity protection  |

|                                 info            |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP PREPARE SUB HANDOVER  RELOCATION REQUEST ACK|

result  | response                                        |

| -an-APDU(                                       |

|  HANDOVER REQUEST ACK)                          |

|                                                 |

| BSSMAP information      RANAP information       |

|      elements:              elements:           |

|                                                 |

|   Layer 3 info            tRNC to sRNC container|

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| MAP SUB PREPARE HANDOVER  RELOCATION FAILURE    |

result  | response                                        |

| -an-APDU(                                       |

|  HANDOVER FAILURE)                              |

|                                                 |

The interworking between HANDOVER FAILURE and MAP Process Signalling  Request in 3G_MSC-B is as follows:

----------------------------------------------------------------

|   08.08                          29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | HANDOVER FAILURE         MAP PROCESS-SIGNALLING |

message |                                   request       |

|                          -an-APDU(              |

|                           HANDOVER FAILURE)     |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

|                                                 |

The interworking between Send End Signal Response and RELOCATION COMPLETE in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION COMPLETE  MAP SEND END SIGNAL        |

message |                               response          |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                          MAP U/P -ABORT         |  1

|                                                 |

NOTE 1:	The abortion of the dialogue ends the handover procedure with MSC-B.

### 4.7.3	Subsequent Inter-MSC Handover to third MSC

When a Mobile Station is being handed over to a third MSC, the procedure (described in 3GPP TS 23.009) does require one specific interworking case in MSC-A (figure 49) between E-Interface from MSC-B and E-Interface from 3G_MSC-B' other than the combination of the ones described in the subclause 4.5.1 and 4.7.2.

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |

|----------->|MAP PREPARE SUBSEQUENT|                |

|REQUIRED    |--------------------->|                |

|            |HANDOVER request      |MAP PREPARE     |

|            |                      |--------------->|

|            |                      |HANDOVER request|

|            |                      |                |+-------+

|            |                      |                ||Possib.|

|            |                      |                ||Alloc. |

|            |                      |                ||of ho. |

|            |                      |                ||number |

|            |                      |                || VLR-B |

|            |                      |                |+-------+

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |-------->|

|            |                      |                |REQUEST  |

|            |                      |                |

Figure 45: Signalling for Subsequent Inter-MSC Handover to third MSC (3G_MSC-B') initiation

Possible Positive outcomes: successful radio resources allocation:

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |                |LOCATION |

|            |                      |                |-------->|

|            |                      |                |REPORTING|

|            |                      |                |CONTROL  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|    response    |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

|            |                      |                |         |

Figure 46: Signalling for Subsequent Inter-MSC Handover to third MSC (3G_MSC-B') execution
(Positive outcome)

Possible Negative outcomes:

a)	user error detected, or component rejection or dialogue abortion performed by MSC-B':

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |response negative result  |

|            |                      |MAP CLOSE       |         |

|            |                      |<---------------|         |

|            |                      |MAP U/P -ABORT  |         |

|            |MAP PREPARE SUBSEQUENT|                |         |

|            |<---------------------|                |         |

|            |HANDOVER response negative             |         |

| HANDOVER   |result                |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

b)	radio resources allocation failure:

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| HANDOVER   |                      |                |         |

|<-----------|                      |                |         |

| REQUIRED   |                      |                |         |

| REJECT     |                      |                |         |

| (Note 1)   |                      |                |         |

Figure 47: Signalling for Subsequent Inter-MSC Handover to third MSC (3G_MSC-B') execution (Negative outcome)

NOTE 1:	Possible rejection of the handover because of the negative outcome of MAP or BSSMAP procedure.

Positive outcome:

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |COMPLETE |

|            |                      |                |         |

|            |                      |MAP SEND END SIGNAL       |

|            |                      |<---------------|         |

|            | MAP SEND END SIGNAL  |request         |         |

|            |<---------------------|                |         |

|            | response             |                |         |

| CLEAR      |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 48: Signalling for Subsequent Inter-MSC Handover to third MSC (3G_MSC-B') completion (Successful completion of the procedure)

Negative outcome:

BSS-A      MSC-B                  MSC-A           3G-MSC-B'

|            |                      |                |

|HANDOVER    |                      |                |    RNS-B'

|----------->|                      |                |         |

|FAILURE     |MAP PROCESS ACCESS    |                |         |

|            |--------------------->|                |         |

|            |SIGNALLING request (Note 1)            |         |

|            |                      |                |         |

|            |                      |MAP U -ABORT    |         |

|            |                      |--------------->|         |

|            |                      |                |IU RELEASE

|            |                      |                |-------->|

|            |                      |                |COMMAND  |

|            |                      |                |         |

Figure 49: Signalling for Subsequent Inter-MSC Handover to third MSC (3G_MSC-B') completion (Unsuccessful completion of the procedure)

NOTE:	Specific interworking case detailed below.

BSS-A         MSC-A                   3G-MSC-B'             RNS-B'

|               |                         |                   |

|               |                         |LOCATION REPORT    |

|               |                         |<------------------|

|               |MAP PROCESS ACCESS       |                   |

|               |<------------------------|                   |

|               |SIGNALLING               |                   |

Figure yy: Signalling for updating of anchor MSC after change of location in RNS

The specific interworking case in MSC-A compared to the subclauses 4.5.1 and 4.7.2 occurs between HANDOVER FAILURE encapsulated in a Process Access Signalling from MSC-B and the abortion of the dialogue with 3G_MSC-B' in the case of a reversion to old channel of the MS:

----------------------------------------------------------------

|   29.002                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PROCESS-SIGNALLING                          |

message |         request                                 |

|                                                 |

|         -an-APDU(                MAP U -ABORT   |  1

|         HANDOVER FAILURE)                       |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                  MAP U/P -ABORT |  2

|                                                 |

NOTE 1:	The abortion of the dialogue triggers in 3G_MSC-B' the clearing of the circuit connection with MSC-A, if any, and of the Resources between 3G_MSC-B' and RNS-B'. The abortion of the dialogue ends the handover procedure with 3G_MSC-B'.

NOTE 2:	The abortion of the dialogue ends the handover procedure with MSC-B.

### 4.7.4	BSSAP Messages transfer on E-Interface

The handling is described in chapter 4.5.4, additional cases are described in this chapter.

#### 4.7.4.1	Assignment

The interworking between the BSSMAP assignment messages in MAP and the RANAP RAB assignment messages is as follows:

----------------------------------------------------------------

|    29.002                        25.413         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PREPARE HANDOVER          RAB ASSIGNMENT REQ|

message |         request                                 |

|         -an-APDU(                               |

|         ASSIGNMENT REQUEST)                     |

|                                                 |

| BSSMAP information            RANAP information |

|      elements:                    elements:     |

|                                                 |

| Channel Type                  RAB parameters    |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP PREPARE HANDOVER                            |

result  |         request                                 |

|         -an-APDU(                               |

|         ASSIGNMENT COMPLETE   RAB ASSIGNMENT    |

|                                  RESPONSE       |

|         or                     (positive result)|

|         ASSIGNMENT FAILURE)   RAB ASSIGNMENT    |

|                                  RESPONSE       |

|                                (negative result)|

|                                                 |

| BSSMAP information            RANAP information |

|      elements:                    elements:     |

|                                                 |

| Cause                         Cause             |	1

|                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP U/P –ABORT    |

|                                                 |

#### 4.7.4.2	Cipher Mode Control

The interworking between the BSSMAP cipher mode messages in MAP and the RANAP security mode messages is as follows:

----------------------------------------------------------------

|    29.002                        25.413         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP FORWARD ACCESS SIGN.      SECURITY MODE CMD |

message |         request                                 |

|         -an-APDU(                               |

|         CIPHER MODE CMD)                        |

|                                                 |

| BSSMAP information            RANAP information |

|      elements:                    elements:     |

|                                                 |

| Encryption information        Integrity         |

|                                  protection info|

|                               Encryption info   |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP PROCESS ACCESS SIGN.                        |

result  |         request                                 |

|         -an-APDU(                               |

|         CIPHER MODE COMPLETE  SECURITY MODE     |

|         or                       COMPLETE       |

|         CIPHER MODE REJECT)   SECURITY MODE     |

|                                  REJECT         |

|                                                 |

| BSSMAP information            RANAP information |

|      elements:                    elements:     |

|                                                 |

| Encryption information        Integrity         |

|                                  protection info|

|                               Encryption info   |

| Cause                         Cause             |	1

|                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP U/P –ABORT    |

|                                                 |

#### 4.7.4.3	Location Reporting Control

The interworking between the RANAP location report message and the BSSMAP handover performed message in MAP is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | LOCATION REPORT     MAP PROC. ACC. SIGNALLING   |

message |                                                 |

|                              -an-APDU(          |

|                              HANDOVER PERFORMED)|

|                                                 |

| RANAP information            BSSMAP information |

|      elements:                    elements:     |

|                                                 |

| Area identity (SAI)          Cell identifier    |

| Cause                        Cause              |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

### 4.7.5	Processing in 3G_MSC-B, and information transfer on E-interface

The following parameters require processing (e.g. to store the parameter, to internally generate the parameter) in MSC-B. The relevant BSSMAP procedures are mentioned to ease the comprehension, their detailed description is the scope of GSM 08.08. Each BSSMAP message listed in GSM 09.08 being transferred on E-interface shall use the mechanisms given in subclause 4.5.4 and is described in GSM 08.08.

#### 4.7.5.1	Encryption Information

The list of GSM algorithms, the ciphering key and the chosen algorithm shall be stored by 3G_MSC-B and used for generating the UMTS parameters Encryption Information and Integrity Protection Information if they are not received in MAP Prepare Handover Request (the generation of the UMTS parameters from the GSM parameters is described in TS 33.102).

Transfer of Information:

If ciphering has not been performed before Inter-MSC Handover, this will be controlled by MSC-A after the completion of Inter-MSC Handover.

Ciphering control towards 3G_MSC-B:

If Ciphering has been performed before Inter-MSC Handover:

-	in the Handover Request BSSMAP message (information included).

The Handover Request Acknowledge should in this case NOT contain the indication of the chosen algorithm.

If Ciphering has NOT been performed before Inter-MSC Handover:

-	in the Cipher Mode Command procedure between MSC-A and 3G_MSC-B.

#### 4.7.5.2	Channel Type

The Channel Type shall be stored by 3G_MSC-B and used for generating RAB parameters.

Transfer of Information:

Independently of the type of resource (Signalling only or traffic channel) assigned to the MS, the Channel Type Information is transferred to 3G_MSC-B in:

-	the Handover Request BSSMAP message.

Chosen Channel and/or Speech Version shall NOT be reported back to MSC-A in the Handover Request Acknowledge

If a new type of resource is to be assigned after Inter-MSC Handover, this can be made with:

-	the BSSMAP Assignment procedure between MSC-A and 3G_MSC-B.

#### 4.7.5.3	Classmark

This information shall be stored by 3G_MSC-B and might be received from MSC-A.

Transfer of Information due to Classmark received from MSC-A:

This information shall be stored by 3G_MSC-B and is received:

-	in the Handover Request BSSMAP message.

If a new type of resource is to be assigned after Inter-MSC Handover, Classmark Information MAY be included:

-	in the BSSMAP Assignment procedure.

#### 4.7.5.4	Priority

The parameter shall be stored by 3G_MSC-B and used for generating RAB parameters. It is received as detailed below:

Transfer of Information:

Received by 3G_MSC-B from MSC-A in:

-	the Handover Request BSSMAP message.

If a change is needed after an Inter-MSC Handover with:

-	the BSSMAP Assignment procedure.

#### 4.7.5.5	MSC-Invoke Trace Information Elements

The process to be performed by 3G_MSC-B on the information elements of the MSC Invoke Trace BSSMAP messages is left for further study.

Note that MSC-A does not forward BSC Invoke Trace in case of GSM to UMTS handover.

### Cause Code Mapping

When a Mobile Station is handed over between GSM and UMTS, a mapping of the cause codes used in the BSSMAP and the RANAP protocols is needed. The mapping  described here is applicable to the BSSMAP protocol even when used inside MAP in the E-interface.

The mapping between the cause codes received in BSSMAP Handover Required and the cause codes sent in RANAP Relocation Request is as follows:

----------------------------------------------------------------

08.08                         25.413              |Notes

-------------------------------------------------------┼--------

HANDOVER REQUIRED              RELOCATION REQUEST      |

|

-Better Cell                   -Time critical reloc.  |

-Directed retry                -Directed retry        |

-Distance                      -Time critical reloc.  |

-Downlink quality              -Time critical reloc.  |

-Downlink strength             -Time critical reloc.  |

-O and M intervention          -O and M intervention  |

-Preemption                    -RAB pre-empted        |

-Response to MSC invocation    -Time critical reloc.  |

-Switch circuit pool                                  |	 1

-Traffic                       -Time critical reloc.  |

-Uplink quality                -Time critical reloc.  |

-Uplink strength               -Time critical reloc.  |

-Any other value               -Time critical reloc.  |

NOTE 1: Cause code not used at inter-system handover.

The mapping between the cause codes received in BSSMAP Handover Request and the cause codes sent in RANAP Relocation Request is as follows (the mapping is only used for the MAP-E interface):

----------------------------------------------------------------

08.08                         25.413              |Notes

-------------------------------------------------------┼--------

HANDOVER REQUEST               RELOCATION REQUEST      |

|

-Better Cell                   -Time critical reloc.  |

-Directed retry                - Directed retry       |

-Distance                      -Time critical reloc.  |

-Downlink quality              -Time critical reloc.  |

-Downlink strength             -Time critical reloc.  |

-O and M intervention          -O and M intervention  |

-Preemption                    -RAB pre-empted        |

-Response to MSC invocation    -Time critical reloc.  |

-Switch circuit pool                                  |	 1

-Traffic                       -Time critical reloc.  |

-Uplink quality                -Time critical reloc.  |

-Uplink strength               -Time critical reloc.  |

-Any other value               -Time critical reloc.  |

NOTE 1: Cause code not used at inter-system handover.

The mapping between the cause codes received in BSSMAP Handover Failure and the cause codes sent in RANAP Iu Release Command is as follows:

----------------------------------------------------------------

08.08                         25.413              |Notes

-------------------------------------------------------┼--------

HANDOVER FAILURE               IU RELEASE COMMAND      |

|

-Ciphering algorithm not                              |	 2

supported                                            |

-Circuit pool mismatch                                |	 1

-Equipment failure             -Relocation cancelled  |

-Invalid message contents      -Abstract Syntax Error |

-No radio resource available                          |	 2

-O and M intervention          -O and M intervention  |

-Radio interface failure,      -Relocation cancelled  |

reversion to old channel                             |

-Radio interface message       -Relocation cancelled  |

failure                                              |

-Requested speech version                             |	 2

unavailable                                          |

-Requested terrestrial                                |	 2

resource unavailable                                 |

-Requested transcoding/rate                           |	 2

adaption unavailable                                 |

-Switch circuit pool                                  |	 1

-Terrestrial circuit already   -Relocation cancelled  |

allocated                                            |

-Any other value               -Relocation cancelled  |

NOTE 1: Cause code not used at inter-system handover.

NOTE 2: Cause code not applicable to this traffic case.

The mapping between the cause codes received in RANAP Relocation Failure and the cause codes sent in BSSMAP Handover Failure is as follows (this mapping is only used for the MAP-E interface):

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

RELOCATION FAILURE             HANDOVER FAILURE        |

|

-Any value                      -No radio resource     |

available             |

The mapping between the cause codes received in RANAP Relocation Failure and the cause codes sent in BSSMAP Handover Request Reject is as follows:

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

RELOCATION FAILURE     HANDOVER REQUIRED REJECT        |

|

-Any value                      -No radio resource     |

available             |

The mapping between the RANAP and the BSSMAP assignment messages is used in the MAP-E interface. RANAP RAB Assignment Response with successful result is mapped to BSSMAP Assignment Complete; RANAP RAB Assignment Response with unsuccessful result is mapped to BSSMAP Assignment Failure. The mapping between the cause codes received in RANAP RAB Assignment Response and the cause codes sent in BSSMAP Assignment Failure is as follows (this mapping is only used for the MAP-E interface):

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

RAB ASSIGNMENT RESPONSE        ASSIGNMENT FAILURE      |

|

-Requested traffic class not   –No radio resource      |

available                      available              |

-Invalid RAB parameters value  –Invalid msg. contents  |

-Requested max bit rate not    –No radio resource      |

available                      available              |

-Requested max bit rate for DL –No radio resource      |

not available                  available              |

-Requested max bit rate for UL –No radio resource      |

not available                  available              |

-Requested guaranteed bit rate –No radio resource      |

not available                  available              |

-Requested guaranteed bit rate –No radio resource      |

for DL not available           available              |

-Requested guaranteed bit rate –No radio resource      |

for UL not available           available              |

-Requested transfer delay not  –No radio resource      |

achievable                     available              |

-Invalid RAB param. combination–Invalid msg. contents  |

-Condition violation for SDU   –Invalid msg. contents  |

parameters                                            |

-Condition violation for       –Invalid msg. contents  |

traffic handling priority                             |

-Condition violation for       –Invalid msg. contents  |

guaranteed bit rate                                   |

-User plane not supported      –No radio resource      |

available              |

-Iu UP failure                 –Equipment failure      |

-Tqueuing expiry               –Radio interface message|

failure                |

-Invalid RAB id                –Invalid msg. contents  |

-Request superseeded           –No radio resource      |

available              |

-Relocation triggered          -No radio resource      |

available              |

-Any other value               –Radio interface message|

failure                |

The mapping between the cause codes received in RANAP Location Report and the cause codes sent in BSSMAP Handover Performed is as follows (this mapping is only used for the MAP-E interface):

----------------------------------------------------------------

25.413                         08.08              |Notes

-------------------------------------------------------┼--------

LOCATION REPORT                HANDOVER PERFORMED      |

|

-User restriction start ind.   –O&M intervention       |

-User restriction start ind.   –O&M intervention       |

-Requested report type not                             |  1

supported                                             |

-Any other value               -Better cell            |

NOTE 1: In this case, no Handover Performed is sent.

## 4.8	Inter-MSC Relocation

The general principles of the relocation procedures are given in Technical Specification TS 23.009. TS 29.010 gives the necessary information for interworking between the TS 25.413 relocation protocol and the TS 29.002 MAP protocol.

For intra UMTS handovers, RANAP is carried over the MAP-E interface instead of BSSAP. Please refer to 3GPP TS 29.108.

### 4.8.1	Basic Inter-MSC Relocation

When a Mobile Station is relocated between two MSCs, the establishment of a connection between them (described in TS 23.009) requires interworking between Iu-Interface and E-Interface.

The signalling at initiation, execution and completion of the Basic Inter-MSC relocation procedure is shown in figures 50 to 54 with both possible positive or negative outcomes.

Additionally figure 50b shows the possible interworking when trace related messages are transparently transferred on the E-Interface at Basic Inter-MSC Relocation initiation.

RNS-A        3G-MSC-A                  3G-MSC-B

|               |                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request                  | |Possible Alloc. |

|               |                         | |of a relocation|

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

Figure 50a: Signalling for Basic Inter-MSC Relocation initiation (no trace related messages transferred)

RNS-A        3G-MSC-A                  3G-MSC-B

|             (*)                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE HANDOVER     |

|REQUIRED       |------------------------>| +----------------+

|               |request  (**)            | |Possible Alloc. |

|               |                         | |of a relocation |

|               |                         | |no. in the VLR-B|

|               |                         | +----------------+

|               |                         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

|               |                         |                   |

|               |                         |CN INVOKE TRACE    |

|               |                         |--------------->(***)

Figure 50b: Signalling for Basic Inter-MSC Relocation initiation (CN invoke trace message transferred)

(*):	Tracing invocation has been received from VLR.

(**):	In that case, RELOCATION REQUEST and CN INVOKE TRACE messages are included within the AN-apdu parameter.

(***):	CN INVOKE TRACE is forwarded to RNS-B if supported by 3G_MSC-B.

Possible Positive outcomes: successful radio resources allocation and relocation numbers allocation (if performed):

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE HANDOVER    |                   |

|               |<------------------------|                   |

|               | response                |                   |

|               |                         |                   |

|RELOCATION COMMAND                       |                   |

|<--------------|                         |                   |

Possible Negative outcomes:

a)	user error detected, or relocation numbers allocation unsuccessful (if performed), or component rejection or dialogue abortion performed by 3G_MSC-B:

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |MAP PREPARE HANDOVER response                |

|               |negative result, MAP CLOSE                   |

|               |<------------------------|                   |

|               |MAP U/P-ABORT            |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE        |       MAP CLOSE         |                   |

|               |------------------------>|                   |

|               |                         |                   |

b)	radio resources allocation failure:

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |                         |RELOCATION FAILURE |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |                   |

|               |<------------------------|                   |

|               | response                |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE        |                         |                   |

|               |                         |                   |

c)	radio resources allocation partial failure (3G_MSC-A decides to reject the relocation):

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |<------------------|

|               |MAP PREPARE HANDOVER     |      ACK          |

|               |<------------------------|                   |

|               | response                |                   |

|RELOCATION PREPARATION                   |                   |

|<--------------|                         |                   |

|FAILURE        |                         |                   |

|               |                         |                   |

d)	unsuccessful relocation execution (relocation cancelled):

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|RELOCATION     |                         |                   |

|-------------->|                         |                   |

|CANCEL         |                         |                   |

|               |MAP U -ABORT             |                   |

|               |------------------------>|                   |

|               |                         |IU RELEASE COMMAND |

|RELOCATION     |                         |------------------>|

|<--------------|                         |                   |

|CANCEL ACK     |                         |                   |

|               |                         |                   |

Figure 51: Signalling for Basic Inter-MSC Relocation execution (Negative outcomes)

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |                         |RELOCATION COMPLETE|

|               |                         |<------------------|

|               |MAP SEND END SIGNAL request                  |

|               |<------------------------|                   |

|IU RELEASE COMMAND                       |                   |

|<--------------|                         |                   |

|               |                         |                   |

Figure 52: Signalling for Basic Inter-MSC Relocation completion

Positive outcome

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               |MAP SEND END SIGNAL      |                   |

|               |------------------------>|                   |

|               | response                |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         | (Note 1)          |

Figure 53: Signalling for Basic Inter-MSC Relocation completion (Positive outcome)

NOTE:	From interworking between MAP and RANAP point of view.

Negative outcome:

RNS-A       3G-MSC-A                   3G-MSC-B            RNS-B

|               |                         |                   |

|               | MAP U/P -ABORT          |                   |

|               |------------------------>|                   |

|               |                         |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |                   |

Figure 54: Signalling for Basic Inter-MSC Relocation completion (Negative outcome)

The relocation procedure is normally triggered by RNS-A by sending a RELOCATION REQUIRED message on Iu-Interface to 3G_MSC-A. The invocation of the Basic Inter-MSC relocation procedure is performed and controlled by 3G_MSC-A. The sending of the MAP Prepare-Handover request to 3G_MSC-B is triggered in 3G_MSC-A upon receipt of the RELOCATION REQUIRED message. The identity of the target RNC where the call is to be handed over in 3G_MSC-B area, provided in the RELOCATION REQUIRED message, is mapped to the target RNC Id MAP parameter and the RELOCATION REQUEST message is encapsulated in the an-APDU MAP parameter of the Prepare-Handover MAP request. 3G_MSC-B can invoke another operation towards the VLR-B (allocation of the relocation numbers described in 3GPP TS 29.002).

Additionally, if tracing activity has been invoked, the trace related messages can be transferred on the E-Interface encapsulated in the an-APDU MAP parameter of the Prepare-Handover Request. If transferred, one complete trace related message at a time shall be included in the an-APDU MAP parameter after the RELOCATION REQUEST message.

The interworking between Prepare Handover and RELOCATION REQUIRED is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION REQUIRED MAP PREPARE HANDOVER request|

message |                                                 |

|                            -ho-NumberNotRequired|   1

|                            -target RNC Id       |

|  RANAP information         -Radio Resource Info |

|         elements           -an-APDU(            |

|                            RELOCATION REQUEST,  |   2

|                            CN INVOKE TRACE)     |

--------┼-------------------------------------------------┼-----

Positive|      MAP PREPARE HANDOVER response              |

result  |                                                 |   3

|                             -relocation numbers |

|                             -an-APDU(           |

| RELOCATION COMMAND           RELOCATION REQUEST |

|                              ACKNOWLEDGE        |

|                              or                 |

| RELOCATION PREP FAILURE      RELOCATION FAILURE)|

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| RELOCATION PREP FAILURE     MAP PREPARE HANDOVER|

result  |                                                 |

| Unspecified failure          System Failure     |

| Unspecified failure          No Handover Number |

|                                 available       |

| Unspecified failure          UnexpectedDataValue|

| Unspecified failure          Data Missing       |

|                                                 |

| Unspecified failure          MAP CLOSE          |

| Unspecified failure          MAP U/P -ABORT     |

|                                                 |

NOTE 1:	The RANAP information elements are already stored in 3G_MSC.

The ho-NumberNotRequired parameter is included by 3G_MSC-A, when 3G_MSC-A decides not to use any circuit connection with 3G_MSC-B. No relocation numbers shall be present in the positive result. Any negative response from 3G_MSC-B shall not be due to relocation number allocation problem.

NOTE 2:	The process performed on the RANAP information elements received in the RELOCATION REQUIRED message is described in the 3GPP TS 25.413.

NOTE 3:	The response to the Prepare-Handover request can include in its an-APDU parameter, identifying the 3GPP TS 25.413 protocol, either a RANAP RELOCATION REQUEST ACKNOWLEDGE or a RANAP RELOCATION FAILURE.

In the first case, the positive result triggers in 3G_MSC-A the sending on Iu-Interface of the RELOCATION CMD.

In the second case, the positive result triggers in 3G_MSC-A the sending of the RELOCATION PREP FAILURE.

The interworking between Send End Signal and RELOCATION COMPLETE in 3G_MSC-B is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION COMPLETE MAP SEND END SIGNAL request |

message |                                                 |

|                               -an-APDU(         |

|                               RELOCATION COMPL) |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| IU RELEASE COMMAND  MAP SEND END SIGNAL response|

result  |  -Normal release                                |  1

--------┼-------------------------------------------------┼-----

Negative| IU RELEASE COMMAND                              |

result  |  -Normal release              MAP CLOSE         |  2

|  -Normal release              MAP U/P -ABORT    |

|                                                 |

NOTE 1:	The positive empty result triggers the clearing of the Radio Resources on the Iu-Interface and the release of the SCCP connection between 3G_MSC-B and RNS-B. If a circuit connection is used between 3G_MSC-A and 3G_MSC-B, the 'Normal release' clearing cause shall only be given to RNS-B when 3G_MSC-B has received a clearing indication on its circuit connection with 3G_MSC-A.

NOTE 2:	The abortion of the dialogue or the rejection of the component triggers in 3G_MSC-B the clearing of its circuit connection with 3G_MSC-A, if any, of the Radio Resources on the Iu-Interface and the release of the SCCP connection between 3G_MSC-B and RNS-B.

The interworking between Send End Signal and IU RELEASE COMMAND in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   29.002                          25.413        |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP SEND END SIGNAL          IU RELEASE COMMAND |

message | request                                         |

|          -an-APDU(            - Successful      |

|          RELOCATION COMPLETE)      Relocation   |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

The interworking between RELOCATION CANCEL in case of relocation cancelled and User Abort in 3G-MSC-A is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION CANCEL               MAP U -ABORT    |

message |                                                 |

|  - Relocation                                   |

|    cancelled                                    |

--------┼-------------------------------------------------┼-----

Positive| RELOCATION CANCEL ACKNOWLEDGEMENT               |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

### 4.8.2	Subsequent Inter-MSC Relocation back to 3G_MSC-A

When a Mobile Station is being relocated back to 3G_MSC-A, the procedure (described in TS 23.009) requires interworking between Iu-Interface and E-Interface.

The signalling at initiation, execution and completion of the Subsequent Inter-MSC relocation procedure is shown in figures 55 to 59.

RNS-A        3G-MSC-B                  3G-MSC-A

|               |                         |

|RELOCATION     |                         |

|-------------->|MAP PREPARE SUBSEQUENT   |

|REQUIRED       |------------------------>|

|               |HANDOVER request         |

|               |                         |                RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |------------------>|

Figure 55: Signalling for Subsequent Inter-MSC Relocation back to 3G_MSC-A initiation

Possible Positive outcomes: successful radio resources allocation:

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |                         |                   |

|               |                         |RELOCATION REQUEST |

|               |                         |<------------------|

|               |                         |ACKNOWLEDGE        |

|               | MAP PREPARE SUBSEQUENT  |                   |

|               |<------------------------|                   |

|               | HANDOVER response       |                   |

|RELOCATION COMMAND                       |                   |

|<--------------|                         |                   |

Figure 56: Signalling for Subsequent Inter-MSC Relocation back to 3G_MSC-A execution (Positive outcome)

Possible Negative outcomes:

a)	user error detected, or component rejection or dialogue abortion performed by 3G_MSC-A:

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |MAP PREPARE SUBSEQUENT HANDOVER              |

|               |<------------------------|                   |

|RELOCATION PREP|   response negative result                  |

|<--------------|                         |                   |

|FAILURE (Note 1)                         |                   |

|               |                         |                   |

b)	component rejection or dialogue abortion performed by 3G_MSC-A:

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |MAP CLOSE, MAP U/P ABORT |                   |

|               |<------------------------|                   |

|IU RELEASE COMMAND                       |                   |

|<--------------|                         |                   |

|               |                         |                   |

c)	radio resources allocation failure:

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |                         | RELOCATION FAILURE|

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |                   |

|               |<------------------------|                   |

|RELOCATION PREP|    HANDOVER response    |                   |

|<--------------|                         |                   |

|FAILURE        |                         |                   |

d)	radio resources allocation partial failure (3G_MSC-A decides to reject the relocation):

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |                         | RELOCATION REQUEST|

|               |                         |<------------------|

|               |MAP PREPARE SUBSEQUENT   |     ACK           |

|               |<------------------------|                   |

|RELOCATION PREP|    HANDOVER response    |                   |

|<--------------|                         |                   |

|FAILURE        |                         |                   |

e)	unsuccessful relocation execution (relocation cancelled):

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|RELOCATION     |                         |                   |

|-------------->|                         |                   |

|CANCEL         |MAP PROCESS ACCESS       |                   |

|               |------------------------>|                   |

|               |   SIGNALLING request    |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |IU RELEASE COMPLETE|

|               |                         |<------------------|

|               |MAP FORWARD ACCESS       |                   |

|               |<------------------------|                   |

| RELOCATION    |   SIGNALLING request    |                   |

|<--------------|                         |                   |

|CANCEL ACK     |                         |                   |

Figure 57: Signalling for Subsequent Inter-MSC Relocation back to 3G_MSC-A execution
(Negative outcome)

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |                         |                   |

|               |                         |RELOCATION         |

|               |MAP SEND END SIGNAL      |<------------------|

|               |<------------------------|COMPLETE           |

|               | response                |                   |

|IU RELEASE CMD |                         |                   |

|<--------------|                         |                   |

Figure 58: Signalling for Subsequent Inter-MSC Relocation back to 3G_MSC-A completion
(Successful completion of the procedure)

NOTE:	Positive outcome case shown in figure 53.

RNS-A       3G-MSC-B                  3G-MSC-A             RNS-B

|               |                         |                   |

|               |MAP U/P -ABORT           |                   |

|               |<------------------------|                   |

|               |                         |IU RELEASE COMMAND |

|               |                         |------------------>|

|               |                         |(Note 1)           |

Figure 59: Signalling for Subsequent Inter-MSC Relocation back to 3G_MSC-A completion (Unsuccessful completion of the procedure)

NOTE:	Abnormal end of the procedure that triggers the clearing of all resources in 3G_MSC-B.

The interworking between Prepare Subsequent Handover and RELOCATION REQUIRED is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward |REL. REQUIRED     MAP PREPARE SUBSEQUENT HANDOVER|

message |                              request            |

|                                                 |

|                              -target MSC number |

|                              -target RNC Id     |

| RANAP information            -an-APDU(          |	1

|        elements               RELOCATION REQ)   |

|                                                 |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                  MAP PREPARE SUBSEQUENT HANDOVER|

result  |                             response            |  2

|                             -an-APDU(           |

|RELOCATION CMD.               RELOCATION REQUEST |

|                              ACKNOWLEDGE        |

|                              or                 |

|RELOCATION PREP FAILURE       RELOCATION FAILURE)|

|                                                 |

--------┼-------------------------------------------------┼-----

Negative| REL. PREP. FAILURE        MAP PREPARE SUBSEQUENT|

result  |                           HANDOVER response     |

| Unspecified failure          Unknown MSC        |

| Unspecified failure          Subsequent Handover|

|                                         Failure |

| Unspecified failure          UnexpectedDataValue|

| Unspecified failure          Data Missing       |

|                                                 |

| Iu RELEASE COMMAND           MAP CLOSE          |

|                              MAP U/P –ABORT     |

| Unspecified failure                             |

| Unspecified failure                             |

|                                                 |

NOTE 1:	The processing performed on the RANAP information elements received in the RELOCATION REQUIRED message is out of the scope of the present document. The target MSC number is provided to 3G_MSC-A by 3G_MSB-B based on the information received from RNS-B.

NOTE 2:	The response to the Prepare-Subsequent-Handover request can include in its an-APDU parameter, identifying the 3GPP TS 25.413 protocol, a RANAP RELOCATION REQUEST ACKNOWLEDGE or a RANAP RELOCATION FAILURE.

In the first case, the positive result triggers in 3G_MSC-B the sending on Iu-Interface of the RELOCATION COMMAND.

In the second case, the positive result triggers in 3G_MSC-B the sending of the RELOCATION PREPARATION FAILURE.

The interworking between RELOCATION CANCEL and MAP Process Signalling  Request in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   29.002                         25.413         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PROCESS-SIGNALLING       IU RELEASE COMMAND |

message |         request                                 |

| -an-APDU(                                       |

|  RELOCATION CANCEL)                             |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP FORWARD-SIGNALLING       IU RELEASE COMPLETE|

result  |         request                                 |

| -an-APDU(                                       |

|  RELOCATION CANCEL ACK)                         |

|                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

|                                                 |

The interworking between RELOCATION CANCEL and MAP Process Signalling  Request in 3G_MSC-B is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION CANCEL        MAP PROCESS-SIGNALLING |

message |                                   request       |

|                          -an-APDU(              |

|                           RELOCATION CANCEL)    |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| RELOCATION CANCEL ACK    MAP FORWARD-SIGNALLING |

result  |                                   request       |

|                          -an-APDU(              |

|                           RELOCATION CANCEL ACK)|

|                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                                 |

|                                                 |

The interworking between Send End Signal Result and RELOCATION COMPLETE in 3G_MSC-A is as follows:

----------------------------------------------------------------

|   25.413                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | RELOCATION COMPLETE  MAP SEND END SIGNAL        |

message |                               response          |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                          MAP U/P -ABORT         |  1

NOTE:	The abortion of the dialogue ends the relocation procedure with 3G_MSC-B.

### 4.8.3	Subsequent Inter-MSC Relocation to third MSC

When a Mobile Station is being relocated to a third MSC, the procedure (described in 3GPP TS 23.009) does require one specific interworking case in 3G_MSC-A (figure 64) between E-Interface from 3G_MSC-B and E-Interface from 3G_MSC-B' other than the combination of the ones described in the subclause 4.8.1 and 4.8.2.

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|RELOCATION  |                      |                |

|----------->|MAP PREPARE SUBSEQUENT|                |

|REQUIRED    |--------------------->|                |

|            |HANDOVER request      |MAP PREPARE     |

|            |                      |--------------->|

|            |                      |HANDOVER request|

|            |                      |                |+-------+

|            |                      |                ||Possib.|

|            |                      |                ||Alloc. |

|            |                      |                ||of relo|

|            |                      |                ||number |

|            |                      |                || VLR-B |

|            |                      |                |+-------+

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |-------->|

|            |                      |                |REQUEST  |

|            |                      |                |         |

Figure 60: Signalling for Subsequent Inter-MSC Relocation to third MSC (3G_MSC-B') initiation

Possible Positive outcomes: successful radio resources allocation:

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |REQUEST  |

|            |                      |                ACKNOWLEDGE

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT|    response    |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

|            |                      |                |         |

Figure 61: Signalling for Subsequent Inter-MSC Relocation to third MSC (3G_MSC-B') execution
(Positive outcome)

Possible Negative outcomes:

a)	user error detected, or component rejection or dialogue abortion performed by 3G_MSC-B':

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |response negative result  |

|            |                      |MAP CLOSE       |         |

|            |                      |<---------------|         |

|            |                      |MAP U/P -ABORT  |         |

|            |MAP PREPARE SUBSEQUENT|                |         |

|            |<---------------------|                |         |

|            |HANDOVER response negative             |         |

| RELOCATION |result                |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

|            |                      |                |         |

b)	radio resources allocation failure:

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |FAILURE  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

|            |                      |                |         |

c)	radio resources allocation partial failure (3G_MSC-A decides to reject the relocation):

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |REQ ACK  |

|            |                      |                |         |

|            |                      |MAP PREPARE HANDOVER      |

|            |                      |<---------------|         |

|            |MAP PREPARE SUBSEQUENT| response       |         |

|            |<---------------------|                |         |

|            |HANDOVER response     |                |         |

| RELOCATION |                      |                |         |

|<-----------|                      |                |         |

| PREPARATION|                      |                |         |

| FAILURE    |                      |                |         |

|            |                      |                |         |

Figure 62: Signalling for Subsequent Inter-MSC Relocation to third MSC (3G_MSC-B') execution (Negative outcome)

Positive outcome:

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|            |                      |                |    RNS-B'

|            |                      |                |         |

|            |                      |                |RELOCATION

|            |                      |                |<--------|

|            |                      |                |COMPLETE |

|            |                      |                |         |

|            |                      |MAP SEND END SIGNAL       |

|            |                      |<---------------|         |

|            | MAP SEND END SIGNAL  |request         |         |

|            |<---------------------|                |         |

|            | response             |                |         |

| IU RELEASE |                      |                |         |

|<-----------|                      |                |         |

| COMMAND    |                      |                |         |

Figure 63: Signalling for Subsequent Inter-MSC Relocation to third MSC (3G_MSC-B') completion (Successful completion of the procedure)

Negative outcome:

RNS-A     3G-MSC-B               3G-MSC-A         3G-MSC-B'

|            |                      |                |

|RELOCATION  |                      |                |    RNS-B'

|----------->|                      |                |         |

|CANCEL      |MAP PROCESS ACCESS    |                |         |

|            |--------------------->|                |         |

|            |SIGNALLING request (Note 1)            |         |

|            |                      |                |         |

|            |                      |MAP U -ABORT    |         |

|            |                      |--------------->|         |

|            |MAP FORWARD ACCESS    |                |IU RELEASE

|            |<---------------------|                |-------->|

|RELOCATION  | SIGNALLING request   |                |COMMAND  |

|<-----------|                      |                |         |

|CANCEL ACK  |                      |                |         |

|            |                      |                |         |

Figure 64: Signalling for Subsequent Inter-MSC Relocation to third MSC (3G_MSC-B') completion (Unsuccessful completion of the procedure)

NOTE:	Specific interworking case detailed below.

The specific interworking case in 3G_MSC-A compared to the subclauses 4.8.1 and 4.8.2 occurs between RELOCATION FAILURE encapsulated in a Process Access Signalling from 3G_MSC-B and the abortion of the dialogue with 3G_MSC-B' in the case of relocation cancelled:

----------------------------------------------------------------

|   29.002                         29.002         |Notes

--------┼-------------------------------------------------┼-----

Forward | MAP PROCESS-SIGNALLING                          |

message |         request                                 |

|                                                 |

|         -an-APDU(                MAP U -ABORT   |  1

|         RELOCATION CANCEL)                      |

|                                                 |

--------┼-------------------------------------------------┼-----

Positive| MAP FORWARD-SIGNALLING                          |

result  |         request                                 |

|         -an-APDU(                               |

|         RELOCATION CANCEL ACK)                  |

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                                  MAP U/P -ABORT |  2

|                                                 |

NOTE 1:	The abortion of the dialogue triggers in 3G_MSC-B' the clearing of the circuit connection with 3G_MSC-A, if any, and of the Resources between 3G_MSC-B' and RNS-B'. The abortion of the dialogue ends the relocation procedure with 3G_MSC-B'.

NOTE 2:	The abortion of the dialogue ends the relocation procedure with 3G_MSC-B.

### 4.8.4	RANAP Messages transfer on E-Interface

The following mapping applies to the encapsulation performed in 3G_MSC-A.

----------------------------------------------------------------

|   25.413                        29.002          |Notes

--------┼-------------------------------------------------┼-----

Forward | RANAP messages     MAP FORWARD ACCESS SIGNALLING|

message |                               request           |  1

|                                                 |

|                       -an-APDU (RANAP messages) |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |  2

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP CLOSE         |

|                               MAP U/P -ABORT    |

|                                                 |

NOTE 1:	Complete RANAP messages to be sent on 3G_MSC-B - RNS-B interface are embedded into the an-APDU parameter.

NOTE 2:	The Return Result does not apply. If 3G_MSC-B returns a message, this message will arrive in an Invoke: Process Access Signalling.

The following mapping applies to the encapsulation performed in 3G_MSC-B.

----------------------------------------------------------------

|   25.413                        29.002          |Notes

--------┼-------------------------------------------------┼-----

Forward | RANAP messages     MAP PROCESS ACCESS SIGNALLING|

message |                                request          |  1

|                                                 |

|                       -an-APDU (RANAP messages) |

--------┼-------------------------------------------------┼-----

Positive|                                                 |

result  |                                                 |  2

--------┼-------------------------------------------------┼-----

Negative|                                                 |

result  |                               MAP CLOSE         |

| IU RELEASE COMMAND                              |

|                                                 |

| Unspecified failure           MAP U/P -ABORT    |  3

|                                                 |

NOTE 1:	Complete RANAP messages to be sent to 3G_MSC-A are embedded into the an-APDU parameter.

NOTE 2:	The Return Result does not apply. If 3G_MSC-A returns a message, this message will arrive in an Invoke: Forward Access Signalling.

NOTE 3:	The abortion of the dialogue triggers the clearing of the circuit connection with 3G_MSC-A, if any, of the Radio Resources on the Iu-Interface and the release of the SCCP connection between 3G_MSC-B and RNS-B. The clearing of the Radio Resources (the clearing indication received from RNS-B is transmitted to 3G_MSC-A) or the loss of the SCCP connection between 3G_MSC-B and RNS-B, triggers in 3G_MSC-B the abortion of the dialogue on the E-Interface and the clearing of the circuit connection with 3G_MSC-A, if any.

### 4.8.5	Processing in 3G_MSC-B, and information transfer on E-interface

The following parameters require processing (e.g. to store the parameter, to internally generate the parameter) in 3G_MSC-B. The relevant RANAP procedures are mentioned to ease the comprehension, their detailed description is the scope of the TS 25.413. Each RANAP message being transferred on E-interface shall use the mechanisms given in subclause 4.8.4 and is described in TS 25.413.

#### 4.8.5.1	Integrity Protection Information

A sequence of possible integrity protection algorithms can be sent to an RNS in Security Mode Command or Relocation Request. The RNS chooses one of the listed algorithms and reports this back to the 3G_MSC in Security Mode Complete or Relocation Request Acknowledge respectively.

The list of algorithms, the integrity protection key and the chosen algorithm shall be stored by 3G_MSC-B.

Transfer of Information:

If integrity protection has not been performed before Inter-MSC Relocation, this will be controlled by 3G_MSC-A after the completion of Inter-MSC Relocation.

Integrity protection control towards 3G_MSC-B:

If Integrity protection has been performed before Inter-MSC Relocation:

-	in the Relocation Request RANAP message (information included).

The Relocation Request Acknowledge should in this case contain the indication of the chosen algorithm.

If Integrity protection has NOT been performed before Inter-MSC Relocation:

-	in the Security Mode Command procedure between 3G_MSC-A and 3G_MSC-B.

#### 4.8.5.2	Encryption Information

A sequence of possible encryption algorithms can be sent to an RNS in Security Mode Command or Relocation Request. The RNS chooses one of the listed algorithms and reports this back to the 3G_MSC in Security Mode Complete or Relocation Request Acknowledge respectively.

The list of algorithms, the ciphering key and the chosen algorithm shall be stored by 3G_MSC-B, and the chosen value sent to 3G_MSC-A.

Transfer of Information:

If ciphering has not been performed before Inter-MSC Relocation, this will be controlled by 3G_MSC-A after the completion of Inter-MSC Relocation.

Ciphering control towards 3G_MSC-B:

If Ciphering has been performed before Inter-MSC Relocation:

-	in the Relocation Request RANAP message (information included).

The Relocation Request Acknowledge should in this case contain the indication of the chosen algorithm.

If Ciphering has NOT been performed before Inter-MSC Relocation:

-	in the Security Mode Command procedure between 3G_MSC-A and 3G_MSC-B.

#### 4.8.5.3	RAB Parameters

The parameters shall be stored by 3G_MSC-B to be used at internal Relocation in 3G_MSC-B.

Transfer of information:

Received by 3G_MSC-B from 3G_MSC-A in:

- The Relocation Request RANAP message.

If a new type of resource is to be assigned after Inter-MSC Relocation, this can be made with:

- The RAB Assignment Request RANAP message.

#### 4.8.5.4	Channel Type

Channel Type is GSM information that is required in BSSMAP Handover Request and shall be provided by 3G_MSC-A. 3G_MSC-B needs this information in case of an intra-MSC UMTS to GSM handover after an inter-MSC relocation. The Channel Type derived from the Bearer Capability that is available in 3G_MSC-A. This mapping is described in 3GPP TS 27.001. Therefore 3G_MSC-A must provide this information in case of an inter-MSC relocation. The Radio Resource Information IE in the MAP Prepare Handover message refers to the Channel Type GSM information.

Channel Type shall be stored by 3G_MSC-B.

Transfer of information:

Received by 3G_MSC-B from 3G_MSC-A in:

- The Prepare Handover Request MAP message.

### 4.8.6	Overview of the Technical Specifications 3GPP interworking for the Inter-MSC Relocation

╔══════════════════════════════════════════════════════════════════════════════════════════════════╗

║ PSTN/ISDN                                                                                        ║

║               ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                 ║

║               ▒       +---------------+                        ▒                                 ║

║ ==============▒====== | ============= | ===============        ▒                                 ║

║ UMTS          ▒       |               |                        ▒                                 ║

║               ▒       V               V                        ▒                                 ║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒                                                ▒                                 ║

║ ▒  RNS-A          3G-MSC-A        3G-MSC-B         RNS-B       ▒     MS                          ║

║ ▒                                                              ▒                                 ║

║ ▒                 +-------+                                    ▒ +-------+                       ║

║ ▒                 |  C M  |                24.008              ▒ |  C M  |                       ║

║ ▒                 +-------|<------------------------------------>+-------|                       ║

║ ▒ ░░░░░░░░░░░░░░░ |  M M  | ░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░▒  |  M M  |                       ║

║ ▒ ░+------+25.413 +-------| 25.413+-------+ 25.413+-------+  ░▒  +-------|                       ║

║ ▒ ░| R R  |<----->|  R R  |<=====>|  R R  |<----->|  R R  |  ░▒  |  R R  |                       ║

║ ▒ ░+------+       +-------+       +-------+       +-------+  ░▒  +-------+                       ║

║ ▒ ░                                                      ░▒                                  ║

║ ▒ ░                |   |    29.010   |   |                   ░▒                                  ║

║ ▒ ░                |   V             V   |                   ░▒                                  ║

║ ▒ ░                |+-----+ 29.002+-----+|                   ░▒                                  ║

║ ▒ ░                ||MAP/E|<----->|MAP/E||                   ░▒                                  ║

║ ▒ ░                |+-----+       +-----+|             23.009░▒                                  ║

║ ▒ ░░░░░░░░░░░░░░░░ | ░░░░░░░░░░░░░░░░░░░ | ░░░░░░░░░░░░░░░░░░░▒  Remark: The RRC interface and   ║

║ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ V ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒          the link layer          ║

║                   +-----------------------+                              protocols being out of  ║

║                   |       T C A P         |                              the scope of this       ║

║                   +-----------------------+                              Specification are not   ║

║    +------------------------------------------------------+              shown here.             ║

║    |                      S C C P                         |                                      ║

║    +------------------------------------------------------+                                      ║

║    +------------------------------------------------------+                                      ║

║    |                        M T P                         |                                      ║

║    +------------------------------------------------------+                                      ║

╚══════════════════════════════════════════════════════════════════════════════════════════════════╝

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CWTS CN# | Spec | Version | CR | <Phase> | New Version | Subject/Comment |
| Sept 1999 | GSM 09.10 | 7.0.0 |  |  |  | Transferred to 3GPP CN |
| CN#04 | 29.010 |  |  | R99 | 3.0.0 | Approved by mail exploder at CN#04 |
| CN#06 | 29.010 | 3.0.0 | 001 | R99 | 3.1.0 | UMTS / GSM Interworking |
| CN#06 | 29.010 | 3.0.0 | 002 | R99 | 3.1.0 | Addition of LSA Information message |
| CN#07 | 29.010 | 3.1.0 | 003r1 | R99 | 3.2.0 | UMTS / GSM Interworking |
| CN#07 | 29.010 | 3.1.0 | 004r1 | R99 | 3.2.0 | GSM / UMTS Interworking |
| CN#07 | 29.010 | 3.1.0 | 005 | R99 | 3.2.0 | UMTS/UMTS Handover |
| CN#09 | 29.010 | 3.2.0 | 006r1 | R99 | 3.3.0 | Clarification of use of Radio Resource Information |
| CN#09 | 29.010 | 3.2.0 | 007r1 | R99 | 3.3.0 | Corrections and updates to align with current R99 specs |
| CN#10 | 29.010 | 3.3.0 | 008 | R99 | 3.4.0 | GSM to 3G Handover: Location Reporting in 3G_MSC-B |
| CN#10 | 29.010 | 3.3.0 | 009 | R99 | 3.4.0 | GSM to 3G Handover: Chosen IEs in Handover Request Ack |
| CN#10 | 29.010 | 3.3.0 | 010 | R99 | 3.4.0 | GSM to 3G Handover: MAP parameter Target Cell ID |
| CN#10 | 29.010 | 3.3.0 | 011r1 | R99 | 3.4.0 | GSM/UMTS Interworking: Mapping of cause codes |
| CN#11 | 29.010 | 3.4.0 | 012 | R99 | 3.5.0 | GSM to UMTS handover: addition of MAP parameter Target RNC ID |
| CN#11 | 29.010 | 3.4.0 | 013 | R99 | 3.5.0 | Inter MSC relocation: addition of MAP parameter Target RNC ID |
| CN#11 | 29.010 | 3.4.0 | 014 | R99 | 3.5.0 | Roaming restrictions for GPRS service |
| CN#11 | 29.010 | 3.4.0 | 015 | R99 | 3.5.0 | Alignment of cause mapping for 08.08 and 25.413 (Directed Retry) |
| CN#11 | 29.010 | 3.4.0 | 016 | R99 | 3.5.0 | UMTS to GSM Directed Retry cause code mapping |
| CN#11 | 29.010 | 3.4.0 | 017 | R99 | 3.5.0 | Mapping of unknown HLR error to access interface cause code |